�����������������������������������������������������������������������������

�����  ���  �   � ����� �     �

�� �   � �   � �     �     �

��    ���� ����� ���   �     �

����� ����� �   � ����� ����� �����

Version 4.0 - November 1995

Copyright 1995

Dan Eble (eble.2@osu.edu)

Magnus Hagander (e95_mha@e.kth.se)

Rob Taylor (100635.152@compuserve.com)

�����������������������������������������������������������������������������

�����������������������������������������������������������������������������

TABLE OF CONTENTS

�����������������������������������������������������������������������������

PART ONE: FOR USERS

-------------------

I. INTRODUCTION

II. REQUIREMENTS

III. PUTTING ZSHELL ON YOUR CALCULATOR

IV. RUNNING ZSHELL

V. RUNNING PROGRAMS FROM ZSHELL

PART TWO: FOR PROGRAMMERS

-------------------------

VI. DIFFICULTIES OF PROGRAMMING THE TI-85

VII. PROGRAMMING FOR THE ZSHELL OPERATING SYSTEM

VIII. LISTS OF HELPFUL INFORMATION

IX. IMPORTANT NEWS IN THIS VERSION!

PART THREE: OTHER TOPICS

------------------------

X.    LEGAL STUFF

XI.   CONTACTING THE DEVELOPERS

XII.  NEW VERSIONS AND INFORMATION

XIII. REVISION HISTORY

�����������������������������������������������������������������������������

PART ONE: FOR USERS

�����������������������������������������������������������������������������

�����������������������������������������������������������������������������

I. INTRODUCTION

�����������������������������������������������������������������������������

ZShell is a program for the TI-85 graphing calculator that enables

execution of Z80-compatible programs stored in string variables.

ZShell is a shell and an operating system combined.  The shell part

provides an organised way to list and execute the string programs stored on

the calculator.  The operating system part provides functions necessary for

the string programs to work properly with different ROM versions and in the

quicksand environment of the TI-85's memory.

ZSHELL40.ZIP

--------------

ZSHELL  .85B -- ZShell 4.0, TexaNoid, Organiser and Function Tester

ZSHELL  .DOC -- this file

TI-85   .H   -- header file for TASM facilitating TI-85 programming.

TASM is the Table Assembler, not Borland's product.

STRING85.COM -- compiles binary Z80 code into an .85S file

UPGRADE.TXT  -- information about how to upgrade old programs (version

3.x) to ZShell 4.0.

TEXAN.DOC    -- Documentation for TexaNoid

ORGANISE.DOC -- Documentation for Organiser

TEST.DOC     -- Documentation for Function Tester

�����������������������������������������������������������������������������

II. REQUIREMENTS

�����������������������������������������������������������������������������

You must have a Graph-Link or know someone who does or know someone who

knows someone who does...

For this version of ZShell, you must have ROM version 3.0A, 4.0, 5.0, 6.0,

8.0, 9.0 or 10.0.  To find out what version of ROM you have, press the

following keys: [2nd] [MODE] [ALPHA] [S].  The sentence "Enter self test?"

accompanied by a version number will appear.  Hit any key EXCEPT [ENTER].

PRESSING [ENTER] WILL CAUSE YOUR MEMORY TO BE ERASED.

�����������������������������������������������������������������������������

III. PUTTING ZSHELL ON YOUR CALCULATOR

�����������������������������������������������������������������������������

ZShell itself is stored in a string variable.  All you have to do to put

it in your calculator is use your the Graph-Link and transfer a memory backup

to your calculator.  To do this without losing whatever is presently in your

memory, follow these steps:

1) Transfer all of the data on your calculator to a group file on the

computer.

2) Transfer the ZShell backup file to your calculator.

3) Transfer the group file back to your calculator.  Don't overwrite any

variables from the ZShell backup.

�����������������������������������������������������������������������������

IV. RUNNING ZSHELL

�����������������������������������������������������������������������������

To run ZSHELL, hit [CUSTOM] [F1].  Note that if you have a ROM version

other than the ones listed in the "REQUIREMENTS" section (II), ZShell will

not run. If you are really unlucky, it might lock your calculator!

NOTE! If the F1 section of the Custom menu does not show "ZShell", DO NOT

PRESS F1, or you calculator will HANG!

Programs made for ZShell 3.0, 2.0 and 1.0 are incompatible with ZShell 4.0

�����������������������������������������������������������������������������

V. RUNNING PROGRAMS FROM ZSHELL

�����������������������������������������������������������������������������

When you start ZShell, it displays a list of the string programs on your

calculator, or "NO PROGRAMS" if it can't find any.  Use the up and down arrow

keys to select a program, [MORE] to see the next page of programs (if you

have more than eight), [ENTER] to run a program, and [EXIT] to return to the

home screen.

�����������������������������������������������������������������������������

PART TWO: FOR PROGRAMMERS

�����������������������������������������������������������������������������

�����������������������������������������������������������������������������

VI. DIFFICULTIES OF PROGRAMMING THE TI-85

�����������������������������������������������������������������������������

1) Different versions of ROM are very much alike, but differ slightly in

the offset of most CALLs.

This also changed with ROM 9.0 - they are not even alike any more....

2) The calculator dynamically rearranges its memory, so there is never

an sure way to pinpoint the location of a program in order to access

data and do JUMPs and CALLs.  One way to circumvent this problem is

to recall a picture variable into the graphics memory (8641).

3) The PIC method of storing programs is convenient, but too inflexible.

It limits (or augments) the size of a program to 1008 bytes and

requires much pressing of keys to go between programs.

�����������������������������������������������������������������������������

VII. PROGRAMMING FOR THE ZSHELL OPERATING SYSTEM

�����������������������������������������������������������������������������

ZShell fixes the ROM CALL offset problem by providing a function that,

given an index, calculates the correct address before calling a ROM routine.

The address of this ROM_CALL routine is 8C09. Here is an example of how

to clear the screen (the clear screen function has been given index 08).

call $8C09   ; this is a call to the ROM_CALL function

.db 08       ; the following byte specify the index

If your assembler can handle macros (I recommend TASM), then you could

define some so that the above looks like this in your code:

ROM_CALL(CLEARLCD)   ; "Ah," you say, "Very nice."

** You can get TASM at ftp.rbk.sollentuna.se in /pub/TI-Calc/software/

** or mrcnext.cso.uiuc.edu in /pub/pcsig as "disk0643.zip"

In order to do CALLs and JUMPs within your own program, there are

functions that do add the program's offset in memory to the address you

supply.  In TI-85.H, we have defined these macros:

; Type of CALL or JUMP      Address

#DEFINE ROM_CALL(index)   call $8C09 \ .db index

#DEFINE CALL_Z(addr)      call $8C0C \ .dw addr

#DEFINE CALL_(addr)       call $8C0F \ .dw addr

#DEFINE CALL_NZ(addr)     call $8C12 \ .dw addr

#DEFINE CALL_C(addr)      call $8C18 \ .dw addr

#DEFINE CALL_NC(addr)     call $8C1E \ .dw addr

#DEFINE JUMP_Z(addr)      call $8C24 \ .dw addr

#DEFINE JUMP_(addr)       call $8C27 \ .dw addr

#DEFINE JUMP_NZ(addr)     call $8C2A \ .dw addr

#DEFINE JUMP_C(addr)      call $8C30 \ .dw addr

#DEFINE JUMP_NC(addr)     call $8C36 \ .dw addr

Therefore, if you wanted to JUMP if the carry flag were set to location

$0ABC relative to the beginning of your program, you would do this:

call $8C30   ; address of JUMP_C

.dw $0ABC    ; address of where to JUMP

Or, with the macros,

JUMP_C($0ABC)

A very simple, size efficient way to do fix the problem.

In order to access data in your program, there are two solutions, one for

temporary data (like players' scores) and one for permanent data (like title

strings).

When using temporary data, the best solution is to use the 168

bytes of the text memory at $80DF.  ZShell automatically sets every byte in

this range to 00 before running any program, so you don't have to worry about

zeroing any matrices yourself, which saves space.

To access permanent data, what you need to do is find your program's

address in memory and add the relative offset of the data.  Fortunately,

ZShell provides the word at ($8C3C), which is the address of the currently

executing program.  For example, if you needed to display the string at

$246 from the start of your program, you would do this:

;    without TI-85.H   ;   with TI-85.H

;

ld hl, ($8C3C)    ;   ld hl, (PROGRAM_ADDR)

ld de, $246       ;   ld de, $246

add hl,de         ;   add hl, de

call $8C09        ; ROM_CALL(D_ZT_STR)   ; display zero-terminated

.db $0B           ;                        ; normal text string

Additionally, ($8C3F) contains the ZShell version number.  It is $40 for

ZShell 4.0.

ZShell 3.1 and later will also recalculate a string's checksum after

it is run, so that permanent variables can be used with little hassle.

To enable this feature, make sure bit 0 of (8C40) is set (1) when your

program terminates.

ZShell 4.0 and later have an option to "warp out". This means that

when the program exits, the user is NOT brought out to ZShell, but

directly to the calculator mode. Useful for "boss-key" and likely.

Here are important ZShell data locations:

PROGRAM_ADDR =$8C3C  ; program address

ROM_VERS	  =$8C3E  ; ROM version byte

ZSHELL_VER	  =$8C3F  ; ZShell version number (3)

ZS_BITS      =$8C40  ; bit 0 set : checksum again

; bit 1 set : warp out

In order to turn your assembled binary code into an .85S, all you have to

do is run it through STRING85.COM.  Here's the batch file I use to assemble,

convert to strings, get ready to transfer my programs to my calculator:

@echo off

tasmb -80 -g3 -q -r8 -y %1.asm   ; assemble source

if errorlevel 1 goto errors      ; stop batch on assembly error

ren %1.obj %1                    ; take extension off object file

string85 %1                      ; convert to string

move %1.85s \link85              ; move to \link85

del %1.bak                       ; delete other files (I have a 40Mb hdd)

del %1.lst

del %1

cd\link85                        ; switch to \link85 and run program

link85

cd\tasm                          ; go back to \tasm

goto done

:errors

echo ERRORS!

:done

From version 4.0, string85.com takes command-line arguments. For

information about these arguments, just type "string85" on the

command-line.

If you are using link85x (and not link85), you should change the

row "move %1.85s \link85" into "move %1.85s \link85\%1.85g".

Of course, you will also have to change the batch-file to suit

your directory structure.

Here are some important points about the structure of your programs:

1) You should use .ORG 0 or your calls and jumps will be messed up.

2) Immediately after .ORG 0, you should define a null-terminated string

to display next to the name in the ZShell menu.

�����������������������������������������������������������������������������

VIII. LISTS OF HELPFUL INFORMATION

�����������������������������������������������������������������������������

The files TI-PORTS.TXT, TI-ROM.TXT, and TI-RAM.TXT contain information

useful and necessary for writing good programs.  These lists are nowhere near

complete (and the RAM list is nowhere near organised :)), so I welcome

corrections and additions.  My Internet address is <eble.2@osu.edu>.

NOTE: The call addresses in TI-ROM.TXT are now obsolete - calling any of these

addresses directly will render program incompatible with many calculators and

could cause them to crash.

There is also a mailing list for ZShell users and developers. Write a

message to "majordomo@kuai.se" with "subscribe list-zshell" in the message

body to get on it.

�����������������������������������������������������������������������������

IX. IMPORTANT NEWS IN THIS VERSION!

�����������������������������������������������������������������������������

From version 4.0 and up, ZShell no longer uses CALL_ROM which took ROM

addresses as arguments It has been replaced by ROM_CALL which takes an index

to a ROM routine. This was required to change in order to include

support for ROM 9.0 and later. This gives the developers the following

problems and solutions:

* Old programs no longer work with ZShell

Solution: Recompile the programs with the new TI-85.H. If you only used

functions declared within it, there should be no problem. If

not, you will have to rewrite it using these functions.

* You used a ROM call not indexed in ZShell 4.0

Solution: You should've told us when we asked developers for ROM calls

used. Your only chance now is mailing us as and ask us to

include it in the next version of ZShell. If we can find a

way to include the routine ROM independent, we will do it.

Alternatively, rewrite the ROM routine yourself. For most of

the ROM routines not catered for in ZShell 4.0 this is a

relatively easy job.

Also note that there are five addresses left in TI-85.H that have not

been exchanged for indexes. This is because they are in fixed location in

ALL ROM versions. Therefor, use a plain call instruction to access them -

no macro or likely defined in TI-85.H.

Note that the FindPixel routine often used (but not declared in TI-85.H)

is NOT to be accessed the usual way. It was completely fixed in location in

ROM <=8.0, but has moved in ROM >= 9.0. Therefor, use the indexed function

FindPixel declared in TI-85.H, and handled by ZShell, instead.

From version 4.0 and up, ZShell will report an error if the user is

trying to execute a program made for a NEWER version of ZShell than the

current. This is used to inform a user that there is a new version out there

somewhere, just waiting to be installed... It will be presented in the same

way as a program with bad checksum, and the program will not be executed.

�����������������������������������������������������������������������������

PART THREE: OTHER TOPICS

�����������������������������������������������������������������������������

�����������������������������������������������������������������������������

X. LEGAL STUFF

�����������������������������������������������������������������������������

ZSHELL.85B and all the companion files are our work.  We encourage their

distribution as long as they are kept together, unmodified from the form in

which we originally distributed them.

You may NOT put ZShell up on any WWW-page, FTP-site or BBS without

agreement with the developers. This is NOT to stop the distribution of

ZShell. It is to allow us to keep track of the distribution sites out there,

so we can make sure a bug-fix or upgrade is always available. Just mail

Magnus Hagander <e95_mha@e.kth.se> for an arrangement.

�����������������������������������������������������������������������������

XI. CONTACTING THE DEVELOPERS

�����������������������������������������������������������������������������

First of all - don't hesitate to contact us. If you have a question that

is about something very basic, that you think a normal user could handle,

use one of the mailing lists. However, if you have some comment or something

more advanced, contact one of us. We will try to respond to all mail sent

to us.

If you want to contact the developers directly, make your first try to

Magnus Hagander (e95_mha@e.kth.se), second to Rob Taylor

(100635.152@compuserve.com) and third to Dan Eble (eble.2@osu.edu). The

reason Dan is so low down on the list is that he is no longer working on

ZShell - he's working on the TI-92 instead.

If you have a question that does not require the assistance from the

developers directly, write to list-zshell. To join list-zshell, write a

message to "majordomo@kuai.se" with "subscribe list-zshell" in the message

body. You will then receive information about how to write to the list etc.

�����������������������������������������������������������������������������

XII. NEW VERSIONS AND INFORMATION

�����������������������������������������������������������������������������

New versions will always be available from:

ftp.rbk.sollentuna.se in /pub/TI-Calc/ti-85/ZShell

as either zshell.zip or zshell<version>.zip. In this directory you can also

find a lot of programs for ZShell. If you have written one of your own,

please upload it here!

New versions will always be announced on the Calc-TI and list-zshell

mailing lists.

A list of authorised mirror-sites are available on ftp.rbk.sollentuna.se

in /pub/TI-Calc/ti-85/ZShell as mirrors.txt. Please use only mirrors listed

in this file. If you on the other hand run a FTP (or WWW) site, and want

to be an authorised mirror site - mail e95_mha@e.kth.se and ask for it.

If you have InterNet and WWW access, please check

http://www.rbk.sollentuna.se/~mha/zshellfan/ - "The ZShell Fan Page".

At this site we collect the thoughts of ZShell users around the world.

Please sign in on this page - it's so nice to know that people appreciate

the work we do.

There is also the "ZShell Homepage", at http://www.kuai.se/~piett/zshell/

On that page you can find up-to-date (hopefully) information on ZShell,

as well as links to ZShell resources etc.

For a reference and examples on the functions defined in TI-85.H, check

http://www2.e.kth.se/~e95_mha/Public/zshell_ref.html.

�����������������������������������������������������������������������������

XIII. REVISION HISTORY

�����������������������������������������������������������������������������

November 1994:

First Release.

January 1995: Version 2.0

Small optimisations.

Cursor returns to last location after program exits.

Cursor wraps from the bottom of the page to the top, and vice versa.

Optional checksum after program termination.

January 1995: Version 3.0

No need to recall ZShell into the graphics screen.

February 1995: Version 3.1

ROM 5.0 support.

November 1995: Version 4.0

ROM 9.0 support. ROM 10.0 support.

Rewrote basic layer for ZShell calls. All earlier programs has to

be recompiled with a new TI-85.H to work with ZShell 4.0, since

ROM offsets has been exchanged for indexes. This caused the

CALL_ROM routine to be renamed to ROM_CALL.

Added ability to "warp" out of ZShell.

Made ZShell inform the user if a program is newer than ZShell.