�

�� ��

����   ��

������     ��           �   �Ŀ �Ŀ �Ŀ ���Ŀ �Ŀ �Ŀ ���

��������      ���         �   �Ĵ � � � � � � � �Ĵ ��� ��

���  �����    ���  ��       ��� � � � � ��� �   � � � ��  � �

�����    ���  �����    ��

�������      ��������      ��    S y s t e m  S p e e d  T e s t

������     �� ������     ��

����   ��     ����   ��             V e r s i o n  6.00

�� ��         �� ��

�             �

Copyright (C) 1993 Landmark Research International Corporation

All Rights Reserved

Valid command-line switches for this program are as follows:

/b  - bypass this initial screen

/nv - don't perform initial video test

/q  - run in quiet mode (no sound)

/xx - run for 'xx' seconds, then exit

USER INTERFACE

You may control this program with a mouse and/or keyboard.

To scroll the text in this box, click on the up and down arrows in the text

box border, or press the up/down arrows, or page up/down keys.

To change the scale of the speed bars (normally yellow), click the left or

right mouse button while the mouse cursor is on top of the bar.  Left button

increases the scale to make the bar shorter (violet), and right button

decreases the scale to make the bar longer (cyan, or light blue).  Press F3

and F4 to increase/decrease the CPU and FPU scales, or F5 and F6 to change

the video scale.

If you are running on a monochrome system, you will not be able to discern

what CPU type or speed scales are active from a distance because color is

used for those purposes.

Press F1 for help, F9 to recalculate video speed, F10 to toggle the beeper

on and off, and Esc to exit the program.

You may run the program with one or more command line parameters separated

by spaces to affect operation:

SPEED /B   - bypass initial help screen

SPEED /NV  - no video (disable video test)

SPEED /Q   - quiet (disable beeper)

SPEED /40  - run for 40 seconds and exit

SPEED /B /NV /Q /40 - all of the above

THE TESTS

The system speed tests tell you the most important information about the

computing performance of your system:  the effective speeds of the central

processor unit (CPU), floating point unit (FPU, math coprocessor),

and video display circuitry.  Although hard disk access and data transfer

rate affect system performance, we excluded those tests because they do not

affect actual computation speed.

CPU/FPU Tests

The CPU and FPU speeds show performance relative to an IBM AT with a 6 MHz

80287 math coprocessor chip installed.  For example, if the display shows

CPU speed to be 22.53 MHz, it means an IBM AT would have to run at 22.53

MHz to have the same computing performance as this computer.  If the display

shows FPU speed to be 40.25 MHz, it means an IBM AT would have to run

its 80287 math chip at 40.25 MHz to have the same floating point math

calculation performance.  The speeds are NOT the actual CPU/FPU clock speeds.

CPU Test

The CPU test forces the processor to show the effect of instruction cache.

It is thus more accurate than previous versions.  On color systems, the

background display is gold for 8088 and 8086 CPUs, green for 80286,

blue for 80386, and red for 80486.  This allows you to see from a distance

what kind of CPU is running the system.  The display also shows in text the

type of CPU installed.

FPU Test

The FPU test shows a value only if a math chip is installed or built into

the CPU.  Where possible, it shows the manufacturer of the chip (e.g., Intel,

Cyrix, IIT).  Note that the 80486 has an internal FPU, but other CPUs do not.

The test accurately reflects the higher performance of a closely coupled

CPU-to-FPU architecture.

Screen Updates

The program runs both CPU and FPU tests continually, and updates the display

about 4 times per second.  The speed will drop if you run another application

concurrently or change CPU clock speed.  You will be able to see the system

speed degradation caused by multitasking or background network accessing,

or by taking the sys

tem in and out of turbo mode.

Clock Speed / Multitasking

Multitasking does not affect actual CPU clock speed, but allows less time

for the speed test to run; turbo mode actually changes the CPU clock speed,

and thus affects the performance of both multitasking and the speed test.

Some systems do not have a turbo mode, but those that do usually allow you

to invoke it with a front panel switch or the keyboard (normally

Ctrl-Alt-plus or Ctrl-Alt-minus).

Variable Readings

As the test runs, you may notice the CPU and FPU MHz values changing

slightly.  This does not mean your system clock is running erratically,

but rather that test results are being rounded off, and occasional

interrupts in the background are using CPU time.

If you want the fastest possible readings, you should boot DOS on the

computer with no drivers or resident programs installed.  Such software can

cause the CPU to work on special tasks in the background while the speed

test runs in the foreground.  The special tasks will take CPU time and cause

the speed test to give lower readings than it should.

Large temperature changes can cause PCs to vary in performance

characteristics.  As a result your computer may run faster when it is cold

than when it is warmed up.

Video Test

The video test checks pure data transfer rate from the CPU into video memory.

It thus shows the maximum speed at which data can be read and displayed.

The video test detects most popular video adapter chipset types, and displays

the type and manufacturer on the screen.  If it cannot determine the

manufacturer, it shows just the generic type, such as monochrome, CGA, EGA,

or VGA.

Video / System Design

The video test results are affected by video adapter design, CPU speed, and

system board design.  For example, some systems have fast I/O bus speeds

and allow a fast data transfers to video adapters.  Others, even some with

very fast CPU speeds, have slow I/O speeds and thus slower display rates.

Much of the power of a fast video adapter can be wasted if it is used in a

slow system.  On the other hand, a slow video adapter can impede the faster

systems, giving diminished performance in display-intensive applications.

Video Runs Once

The program runs the video speed test only once, at the beginning.  This is

partly for aesthetic reasons, since it makes the display look unattractive.

You may re-run the video test by pressing F9 (recalculate).  You will need

to do this if you take the system in or out of turbo mode while the normal

speed test is running, because video speed can be affected by CPU speed.

Video BIOS

The video test does not show the effect of BIOS on display speed because

most popular high-performance applications write directly to video memory,

and do not use BIOS.  Furthermore, most super VGA adapters come with a

program that loads display BIOS into system RAM for maximum execution speed.

SCALES

All of the test results are shown on variable scales.  The default scale

values are different for each CPU type, but the default speed bar is always

yellow.  This allows the program to show how the performance of your

system compares to the scale limits for your processor type.

Thus, if you view the screen from a distance, the program accurately renders

performance information without the exaggerated effect of display wrap-around

or logarithmic scaling used in earlier versions of the program.  If you see a

yellow speed bar, you are seeing the default scaling for your CPU type.

If you are running on a computer that is slow for the class of CPU it uses,

the speed bars will be short.  If the computer is fast, the speed bars will

be longer.  If the bar is too short or too long for your liking, you may

change its length by adjusting the scale.

However, when you change the scale, you will also change the color of the

speed bars to either cyan or violet.  This will allow distant viewers to

recognize that the system is not using the default scale and that the

length of the bars have a different meaning.

If the bar is too long on a scale, you have a very fast computer.  To shorten

it and turn the bar violet, press F3 or click the left mouse button while the

cursor is on top of the bar.  F3 expands CPU and FPU scales, while F5 expands

the video scale.

If the bar is too short on a scale, you are running on a relatively slow

computer.  To lengthen the bar and turn it cyan, press F4 or click the right

mouse button while the cursor is on top of the bar.  F4 shrinks CPU and FPU

scales, while F6 shrinks the video scale.

HISTORY AND STANDARDS

From 1984 through early 1993 the original Landmark CPU Speed Test (tm)

gradually gained prominence and became the de facto world standard for

measuring PC computing speed.  There are several reasons for its rise to

popularity:

1.  Standard - it compared the speed of the computer it ran on to the speed

of the first and only de facto standards that ever existed for personal

computers: the IBM PC and the IBM AT.  IBM never changed the architecture

of either computer, but replaced them with the PS/2 line in 1987 and took

the PC and AT out of production.  Since IBM will never change the PC or AT,

those will always be stable standards of comparison for PC performance,

and Landmark will be the performance measurement standard.  The original PC

used an Intel 8088 CPU and optional 8087 FPU running at 4.77 MHz.  The

original AT used an Intel 80286 CPU running at 6 MHz, and a later version

ran at 8 MHz; in both versions the optional 80287 math chip ran at 6 MHz.

2.  Visibility - the Landmark speed bar that reached horizontally from left

to right across the screen was visible and relatively readable from a

distance of 15 to 30 meters (50 to 100 feet).  This made it ideal for

dealer showrooms and trade shows, so manufacturers and resellers enjoyed

using it to show off their computer's speed.

3.  Credibility - it was the most accurate benchmark test on the market

because it considered a variety of factors that other programs did not, and

it showed performance relative to the PC and AT.  Other benchmark programs

gave unreliable or misleading information:

Norton System Index (SI) was too simple and overrated AT performance relative

to the PC.

MIPS (millions of instructions per second) used an instruction mix that was

somewhat arbitrary.

Whetstone, a test of general computing combined with floating point or

scientific math calculations, was originally written in FORTRAN, so the

results were badly skewed by both compiler efficiency and the ratio of

floating point math instructions to arithmetic and loop instructions.

Dhrystone, a test of general computing performance, was originally developed

to test ADA compiler efficiency, but was converted to the C language for

execution on the PC.  Because of the plethora of C compilers in use, it

suffered from the same kind of skewing as Whetstone.

4.  Results - The speed test displayed its results graphically with fixed

scaling, and it beeped while it gave continuous screen updates.  All the

other programs, such as SI, MIPS, Power Meter, CheckIt, and QA Plus, ran

through once and stopped.  Thus, in real time it showed how much system

performance degraded during network accesses or other multitasking program

activities.  Furthermore, the instant update gave immediate feedback to

users, and gratified them that the system was actively running.  Finally,

benchmark graphic displays in QA Plus and CheckIt automatically scaled test

results so that their speed bars had no meaning when viewed from a distance.

ABOUT LANDMARK

Landmark Research International Corporation is a high-technology research,

development, manufacturing, marketing, and sales organization with

headquarters in Clearwater, Florida.

The original company, Landmark Software, was founded in 1984 in Sunnyvale,

California, to produce the Landmark CPU Speed Test, Landmark Setup program,

and assorted system utility programs.  In early 1990, the company was

restructured with expanded business goals under new leadership and given a

new base of international operations on Florida's west coast.

Landmark develops and supplies computer-related products to many types of

customers:  dealers, distributors, computer manufacturers, 3rd and 4th party

service companies, corporate micro management and repair centers, and

software/hardware developers.

LANDMARK PRODUCTS

Existing products include the following:

1.  The PC Probe (tm) advanced testing program for PCs - it features replete

system information, complete diagnostics, a full suite of benchmark tests

based on the Landmark System Speed Test, CMOS RAM setup and low-level hard

disk format utilities, remote operation, and the ability to display

actual photographs of the circuit or component being tested.  The complete

and perfect program for end users, technicians, dealers, and OEMs to find

out what's in the computer and how well it works.

2.  The AlignIt (tm) maintenance kit for 3.5 and 5.25-inch floppy drives of

all popular densities - it includes a pre-lubricated cleaning diskette

guaranteed for 180 10-second uses, and a patented diagnostic and alignment

spiral diskette that tests and allows drive alignment to within .3 mils.

Perfect for technicians and end users alike to avoid costly drive repairs.

3.  The KickStart (tm) family of diagnostic cards for PCs - the cards assist

technicians in identifying failures in systems that won't boot, and help

Value-Added Resellers to run remote diagnostics on client's computers.

LEDs or digital displays show failing power-on self-test numbers during boot.

Built-in circuitry on some models provides serial, parallel, floppy, and

hard-disk I/O, as well as special test hardware for motherboards.  The design

engineer's and test/repair technician's constant companion.

4.  The Burn (tm) advanced manufacturer's burn-in test for PCs - it features

menu or command line operation via local console or remote terminal, nested

count-based or time-based testing loops, pre-configured user-modifiable test

sequences, and error logging to printer, file, or terminal.  Site licenses

available to PC system and motherboard manufacturers.

5.  The Memory Boss (tm) family of Expanded Memory Managers - fully

compatible with Lotus-Intel-Microsoft Expanded Memory Specification 4.0,

Memory Boss operates on XTs (disk-based emulator), ATs (extended

memory based emulator), 386 ATs (virtual 86 mode based emulator), and

Chips and Technologies NEAT chipset EMS circuitry. Gives Lotus, Windows,

and other applications huge memory space to operate in.  Perfect for

all levels of users; OEM licenses available.

6.  The JumpStart (tm) family of self-configuring motherboard BIOS for 80286,

386, and 486 ISA and EISA computers as well as XTs.  Fully IBM-compatible and

non-infringing.  Available in ROM for system developers and troubleshooters.

Source and object code (label) licenses available for manufacturers.

Other products that have been in development and may be available include

graphic image processing and file conversion; FAX/Data modems with voice mail

software; automated test equipment cards for ATs; and software developer

libraries for XT/AT/386 AT virtual memory management, graphic image data base

management, and menu systems.  Contact Landmark for more information.

When you think of standards, think LANDMARK.

WARRANTY DISCLAIMER

The Author cannot and does not warrant that any functions contained in the

Software will meet your requirements, or that its operations will be error

free.  The entire risk as to the Software performance or quality, or both,

is solely with the user and not the Author.  You assume responsibility for

the selection of the program to achieve your intended results, and for the

installation, use, and results obtained from the Software.

The Author makes no warranty, either implied or expressed, including with-

out limitation any warranty with respect to this Software documented here,

its quality, performance, or fitness for a particular purpose. In no event

shall the Author be liable to you for damages, whether direct or indirect,

incidental, special, or consequential arising out the use of or any defect

in the Software,  even if the Author has been advised of the possibility

of such damages, or for any claim by any other party.  But if you print it

out, light it on fire and your house burns down, don't blame me.

All other warranties of any kind, either express or implied, including but

not limited to the implied warranties of merchantability and fitness for a

particular purpose, are expressly excluded.