This file written 970102 by David Ashley (dash@netcom.com).

This program prints out the header of an EXE file.

When DOS loads a program it decides where in memory the program is going to

be loaded. This means choosing the initial segment in memory. DOS examines

the header to figure out how big the header itself is. The word at offset 8

tells the # of 16 byte paragraphs the header contains. Usually this contains

20h, meaning there are 32 16 byte paragraphs for a total of 512 bytes (200hex).

Call the segment DOS initially loads the program ISEG.

The EXE file after the header gets loaded at segment ISEG+10h, at offset

0 (ISEG+10h:0). The number of bytes that will be loaded is determined by

header words at offsets 4 and 2. Take the word at offset 4, subtract 1, then

multiply by 512. Then add the word at offset 2. This is the total number of

bytes to be loaded.

After loading, the file is relocated if necessary. The word at offset 6 tells

the number of places that need to be relocated. The word at offset 18h tells

where the relocation list is as an offset from the start of the header (which

is also the start of the EXE file).

Each entry in the relocation list is a segment:offset pair. The location to

patch is ISEG+10h+segment:offset. This points to a word that must be fixed.

Dos fixes it by adding the value of ISEG+10h to the contents of the word and

storing the result back into the word's location. That's all there is to it!

The program is ready to run in memory, all dos has to do is set up the

registers and jump to it.

Dos sets up the DS,SS, SP (and I think ES) registers as follows:

DS = ISEG

ES = DS (it always seems to match ds)

SS = ISEG + 10h + word at offset 0eh in the header

SP = word at offset 10h in the header

Now we need to jump into the code. The location is determined by:

CS = ISEG + 10h + word at offset 16h in the header

IP = word at offset 14h in the header

---------------------------

The EXE.C program contains the structure of the header.

Hope this helps.

-Dave