B

RADIO REMOTE CONTROL PROGRAM FOR OS/2

By Lydia Rosa

Copyright 1995,1996  by Lydia Rosa

RADIO REMOTE CONTROL PROGRAM FOR OS/2

CONTENTS

WHAT'S NEW IN THIS RELEASE ....................................... 1

DISCLAIMERS ...................................................... 3

INTRODUCTION ..................................................... 4

REQUIREMENTS ..................................................... 5

STARTUP .......................................................... 7

General Description .............................................. 7

THE DISPLAY ...................................................... 9

VFO Frequency.  .................................................. 9

Modulation.  ..................................................... 9

Channel #.  ...................................................... 9

VFO step value.  ................................................ 10

Use Std Freq.  .................................................. 10

Priority Indicator .............................................. 11

Digit Scrolling ................................................. 11

Freq/Memory Scroll Buttons ...................................... 11

Signal Meter .................................................... 11

Attenuator ...................................................... 12

Scan Mode Buttons.  ............................................. 12

Manual Mode ................................................... 12

Scan Mode ..................................................... 13

Search Mode ................................................... 14

Load and Set Buttons ............................................ 16

Load ALL ...................................................... 16

Set  ALL ...................................................... 16

Mem->VFO ...................................................... 17

VFO->Mem ...................................................... 17

Operation Modification Buttons .................................. 17

COMPUTER/RADIO Button ......................................... 17

VFO Select Button ............................................. 18

Priority ...................................................... 18

Banks On ...................................................... 18

Resume ........................................................ 19

Ack ........................................................... 19

Logging ....................................................... 19

Radio Selection ............................................... 19

Speed Control ................................................. 20

MEMORY  CHANNEL DATA FIELDS ..................................... 20

CHANNEL number ................................................ 20

VALID (Vld) ................................................... 21

DELAY (Dly) ................................................... 21

ATTENUATION (Att) ............................................. 21

RESUME (Rsm) .................................................. 21

FREQUENCY ..................................................... 22

MODE .......................................................... 22

COUNT ......................................................... 22

TIME .......................................................... 22

SIGNAL ........................................................ 22

Contents  ii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SERVICE ....................................................... 23

TIME STAMP .................................................... 23

SCAN BANK  DATA FIELDS .......................................... 23

CHANNEL number ................................................ 23

Valid ......................................................... 24

Start ......................................................... 24

STOP .......................................................... 24

SERVICE ....................................................... 24

SEARCH BANK DATA FIELDS ......................................... 25

CHANNEL number ................................................ 25

VALID ......................................................... 25

Start-Freq .................................................... 26

Stop-Freq ..................................................... 26

Incr-Freq ..................................................... 26

Mode .......................................................... 26

Service ....................................................... 27

Status line ..................................................... 27

SPECTRUM DISPLAY ................................................ 27

The Menu ........................................................ 28

File menu ....................................................... 28

Database menu ................................................... 29

Common Parms panel .............................................. 29

Priority ...................................................... 29

Delay ........................................................ 29

Resume ....................................................... 30

Spectrum Background .......................................... 30

Tone Decode Enable ........................................... 30

Radio  Parms panel .............................................. 30

Radio Name .................................................... 30

Radio Type .................................................... 31

TXADR ......................................................... 31

Max Channel ................................................... 31

Origin .......................................................... 31

Max Frequency ................................................... 31

Min Frequency ................................................... 32

Baud Rate ....................................................... 32

Com Port ........................................................ 32

Data Bits ....................................................... 32

Stop Bits ....................................................... 32

Flow Control .................................................... 32

Signal Detect ................................................... 33

Send Delay ...................................................... 33

SPECIAL KEYS .................................................... 35

ENTER ......................................................... 35

ESC ........................................................... 35

tab keys ...................................................... 35

PgDn/PgUp ..................................................... 35

ctl-PgDn/ctl-PgUp ............................................. 35

alt-PgDn/ctl-PgUp ............................................. 36

DATA FILE FORMATS ............................................... 37

LOGFILE format ................................................ 37

FREQLIST.DBF .................................................. 37

STANDARD.DBF .................................................. 38

Contents  iii

RADIO REMOTE CONTROL PROGRAM FOR OS/2

data file format .............................................. 38

RS232 INTERFACE ................................................. 48

PROGRAM LIMITS AND RANGES ....................................... 49

Contents  iv

RADIO REMOTE CONTROL PROGRAM FOR OS/2

WHAT'S NEW IN THIS RELEASE

Version 0.1

Beta Test Release

Version 0.2

Bug Fix release.

Fixed problem with entering SERVICE data  with signal present.

Fixed several hang problems.

Added VFO attenuation button

Added active HELP display

Added support for AOR8000 (untested)

Version 0.3

Fixed WARP problem with signal strength meter not showing.

Some radio parameters were not being saved in .INI file

Fixed various problems with the KENWOOD protocol.

Increased number of radios to 8.

Added Modem Status Reg (MSR) display.

Added scroll bars for FREQUENCY and MEMORY CHANNEL

Added frequency display sync selection

Fixed spectrum display problems.

Version 0.4

Fixed problem with certain errors not displaying.

Fixed problem with DBASE selection.

Added leading blank removal on Service string load.

Added save/recall of strength values.

enhanced error handling of file read.

added CHAIN command (in TEXT files) for file chaining.

added support for certain commands in TEXT files.

Added support for AOR2500 (untested)

Version 0.5

Added resume for individual memory channels.

Added time stamp for memory channel log.

Modified spectrum display routines.

Fixed sort bug that left displayed channel unsorted.

Version 0.6

Fixed bug -service not being read on SCAN bank.

Fixed bug -SCAN bank info not being saved correctly.

Fixed bug -comments not being rewritten to file.

Fixed bug -KENWOOD modulation not being correctly read.

Fixed bug -Two copies of Radioctl could not run as same time.

bank number now shows current bank.

Added buffer for service info. DBASE file now closed after read.

Added pop-up window on exit for data save.

Added command line option for loading a file.

Version 0.7

Removed UP/DN buttons. Replaced with direct mouse clicks.

Rearranged screen to make room for longer SERVICE strings.

WHAT'S NEW IN THIS RELEASE  1

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Added file logging.

Fixed problem that kept program from working with normal speed UART.

Added support for OptoScan protocol.

Added hardware signal detect for AOR radios.

Added additional PL parms in save file (for future enhancement)

Fixed a few AOR8000 bugs.

Version 0.8

added several pop-up status messages.

Fixed a few file read and write bugs.

Fixed search bug: existing frequency not found in database when

blank channels were present.

Fixed a problem that caused corrupted SERVICE data

when DBASE file had no service information on frequency record.

Version 0.9

Added VFO selection for KENWOOD radios.

Fixed a problem with some KENWOOD radios not being able to SET

channels.

Added support for KENWOOD radios that have signal strength logging.

Fixed memory channel scroll bar so that radio channel changes

when MEMORY tracking mode is selected.

Version 0.91

Added verification screen for SET ALL operation.

Tuned serial port routines for better reliability.

Fixed array bounds problem that cause intermitant failures.

Added VFO selection for ALL radios that support it.

Fixed bug - Hardware/software signal select not working correctly.

Improved AOR8000 routines.

Dropped support for AOR2500 due to difficulties in making this

radio conform to the RadioCtl program. The protocol selection

has been left in, but the routines will most likely not work

correctly.

Version 0.92

Fixed problem with KENWOOD TS870 not setting memory channels.

Version 0.93

Fixed more AOR8000 problems.

Fixed problem with display not being updated during search.

Fixed problem with NEW DBASE menu grayed out if DBASE file not

available at load time.

Version 0.94

Added STANDARD frequency selection for SEARCH.

Cleaned up SEARCH routines for a little speed.

Put some hooks to minimize hanging when a radio is not attached.

Fixed data file write that caused "(null)" written when no service.

WHAT'S NEW IN THIS RELEASE  2

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DISCLAIMERS

IBM is a trademark of IBM corporation.

ICOM is a trademark of Icom incorporated.

KENWOOD is a trademark of Kenwood Corporation.

AOR     is a trademark of AOR inc.

DBASE III is a trademark of Borland International.

RADIO SHACK is a trademark of Tandy Corporation.

The author  takes no  responsibility for  any damages that  may

occur  to your radio or  computer due to  the operation  of  this

program.

This program is being distributed as SHAREWARE. This means that

voluntary  contributions  are  the  only  source of  remuneration

obtained for this work. A contribution of $25.00  gives  you  the

following:

1.  Fixes for any bugs.

2.  Schematics and instructions for the interfaces.

3.  A clear conscience.

Please mail  all contributions, problem reports  and  suggestions

to

Lydia Rosa

P.O. Box 414

Marlboro, N.Y.

12542

The author (and  the  latest versions of  RADIOCTL) can also be

contacted on the following Bulletin Board System:

Rosa's Folly BBS

(914) 236-7607

300/1200/2400/9600/14400 baud

Thank you for your support.

DISCLAIMERS  3

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INTRODUCTION

The  RADIO REMOTE CONTROL PROGRAM for  OS/2  will allow  you to

control selected radios from  any  computer  serial  port (RS232)

using  OS/2.  This program expands the capabilities of your radio

to 10000 channels and  100 programmable scan or search banks.  In

addition, scanning, searching, and logging  facilities  are  also

provided.  Some radios  will  need  an  additional  interface  to

utilize  the  signal  detection  facilities  of  this program.  A

circuit  diagram  and  build instructions  are  provided free  of

charge to all registered users.

The program allows you to select various baud rates, COM ports,

and other parameters  associated with individual  radios.  Up  to

eight different radio configurations  can  be  defined  for  easy

switching.  A  LOCAL mode is also  provided  to  run  the program

without the need for a radio connection.

Frequency information can be loaded  or saved from a text file.

An interface to a DBASE type  file is also provided for frequency

to service lookup. Most setup information is saved in a .INI file

for ease in restarting the program.

INTRODUCTION  4

RADIO REMOTE CONTROL PROGRAM FOR OS/2

REQUIREMENTS

In order to run this program you will need the following:

*    A 386,486 or Pentium based computer.

*    At least 4mb of memory (8mb is recommended).

*    about 1mb of free disk space (for the program and document).

*    OS/2 2.11, WARP, or higher level of OS/2.

*    one of the following models of radio:

-   ICOM R-7000*

-   KENWOOD R-5000*

-   AOR3000a

-   AOR8000

-   RadioShack  radio equipped with OPTOSCAN  interface (from

OptoElectronics).

*NOTE:Although not  tested,  this program  should work with other

models of this manufacturer equipped with a serial port.

For Kenwood and Icom Radios, a modified interface  is  required

for  full program functionally  (signal detection). For all other

radios, the  manufacturer's interface will work.  However, signal

detection can be speeded  up by use of the modified interface for

all of the radios. Plans for  this interface are  made  available

free of charge for all registered users.

It is also recommended  that the COM  driver included with OS/2

be replaced with  the  SIO driver by  Ray Guinn.  This driver has

proved to be more reliable than the default  COM.SYS driver. This

driver  package is  available  in shareware, and can be found  on

most bulletin boards.

If you are controlling the R7000, you will need either the ICOM

CI-V interface adapter, or the  modified RS232 interface.  If you

wish to  use  the signal  detection feature  of this program, you

will need to use the modified interface.

For  the  R-5000,  you  will  need  the  IC-10  option  kit.  A

modification must  be made to the R-5000 if you  wish to  use the

signal  detection feature of  this  program.  Some other  KENWOOD

radios provide  for software signal  detection. Although RADIOCTL

can  make  use of this,  the hardware detection will increase the

spee of scanning.

REQUIREMENTS  5

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The AOR3000,  AOR8000 and OPTO  equipped  radios already have a

RS232 interface. No modification is  needed for these radios,  as

signal  detection  is  done  in software.  However,  use  of  the

modified signal  detection will allow an increase in the speed of

scanning.  In  addition, the modification instructions  include a

change  to the  AOR3000  to allow  9600 baud operation  (from the

default 4800 baud) .

It is highly recommended that  the modified  interface  be used

for ICOM and  KENWOOD  radios.  Most  of  the  features  of  this

program  make  use  of  the  detection  of  an  available  signal

(SQUELCH).  These radios do NOT currently  have squelch detection

as part  of the  standard  interface.  Although this program will

work fine without this  signal detection, operations such as SCAN

and SEARCH will have no usability.

The interface circuit is a  inexpensive, easy to build project.

All  the  parts  are  readily available at any electronic  store.

Estimated cost for each interface  is $15. Plans and instructions

for  these interfaces  are provided  to  you  when  you  become a

registered user.

REQUIREMENTS  6

RADIO REMOTE CONTROL PROGRAM FOR OS/2

STARTUP

It  is  recommended (but not required)  that  this  program  be

installed  in  it's  own  directory.  You  should  also  add  the

directory to your PATH and  DPATH statements  in CONFIG.SYS.  The

FREQLIST.DBF  file should be  placed in the same subdirectory  as

the RADIOCTL.EXE  file. Start the program by entering RADIOCTL on

the command line.  If you  wish, you can  also specify a  file to

load  on the command  line. This file MUST be a  RADIOCTL  format

file  (see  APPENDIX for the  format  of  this  file).  Once  the

program loads, the main control panel  will be displayed. You can

maximize  the window to see all of the  display,  or you can size

the window as you like.

GENERAL DESCRIPTION

RADIOCTL provides the user with a flexible way of controlling a

radio. It also provides a  uniform  interface for different types

of radios. This is a high-level block diagram of the program:

��������Ŀ

|search  |---�

|banks   |   |

����������   |

|

|

|      ������Ŀ

��������Ŀ   �----->|      |   serial  ��������Ŀ

|memory  |<---------|VFO   |---------->|  radio |

|channels|--------->|      |<----------|        |

����������          ��������  �------->|        |

^  ^                      | port   ����������

|  |                      |

|  |    ���������Ŀ       |

|  �----|channel #|-------�

|       �����������

������Ŀ

|scan  |

|banks |

��������

Memory  channels  store  information  on frequency, modulation,

squelch  count, duration, and  other  important  parameters.  The

program  provides  for  a  maximum  of  10000  memory   channels.

Information  can  be  directly  entered  from the control  panel,

loaded  from  a  text  file, loaded from  the  radio's memory, or

transferred from the program's VFO.

The VFO  represents  a  single  frequency and  modulation. This

setting can be sent to the radio, or stored in a  memory channel.

STARTUP  7

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The VFO  can be set from  the control  panel direct input, loaded

from a memory channel, loaded from a search bank, or set from the

radio.

Scan banks provide the ability to group blocks  of channels for

scanning.  Scan  banks  allow  the  user  to  define  up  to  100

different blocks of memory channels for selection.

Search banks provide groups of frequencies to search. Up to 100

different ranges of  frequency, modulation and step  value can be

defined.

The Channel # provides selection of the memory channels. It can

also be used to address the memory channels of the current radio.

Radio control  information  is  sent  to and received from  the

radio  by  way  of  the  serial  port.  The program  provides for

selection of up to eight different radios, each with a unique set

of  operation  characteristics.  The  program  can  also  provide

simulation  of a  radio.  The  actual commands  to the  radio are

defined  by  a  protocol  setting.  The  program current supports

protocols for KENWOOD, ICOM, AOR8000,  AOR3000, and OPTO equipped

radios. A LOCAL protocol is also provided which does not  require

a radio to be attached.

STARTUP  8

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE DISPLAY

The control panel shows you the status  of  your radio, as well

as the status  of the program.  There are  numerous controls, and

various status  fields. Controls are toggled  by moving the mouse

cursor over the control  and  clicking.  Entry  fields  require a

ENTER or TAB key to complete the entry.

VFO FREQUENCY.

This display shows  the current frequency of the program.  This

frequency may or may  not  reflect  the VFO state of the attached

radio.  Any frequency information  sent to the radio is displayed

in this field. The  value of this field can be changed using  the

scroll  buttons to the left of the display or by using  the mouse

buttons when the mouse pointer is over the digit  displayed.  The

left mouse button will  increase  a digit, and  the  right button

will  decrease.  A  frequency  value can also be  entered  in the

entry field under  the  display. If a  value  is entered, changed

with mouse  buttons, or scrolled with  the VFO  scroll bar, it is

also sent to the radio.

MODULATION.

This is  a  combination display and selection box. It shows the

current  modulation  used  by  the program. This may or  may  not

reflect  the state  of the radio. A  modulation  can  be  set  by

pressing the drop-down  control on  the  right  of  the box,  and

double-clicking  on  the desired  modulation. When selected,  the

radio's modulation will be set.

CHANNEL #.

This display shows the current channel number being used by the

program. Depending on the state of the COMPUTER/RADIO  switch, it

will represent the  internal memory  channels  in the program, or

the  radio's  memory channels. The memory channel can be  changed

using the  scroll buttons to the left of the display, using  left

(increment) or  right  (decrement) mouse  buttons,  or entering a

channel  number  in the  entry field  under  the display.  When a

memory channel  number is changed, and  the COMPUTER/RADIO switch

is set to  COMPUTER,  the corresponding computer  memory  channel

information  is  set  to  the  VFO,  and to  the  radio.  If  the

THE DISPLAY  9

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMPUTER/RADIO  switch  is in the  RADIO  position,  the  radio's

memory channel is changed.  If the program is in MANUAL mode, and

the MEM SYNC switch is selected, changing the channel number will

display the corresponding Memory Channel information.

NOTE:The AOR8000 is organized into 20 banks of 50 channels.  Each

bank is identified as a lower or upper case letter. RADIOCTL maps

these channels as follows:

��������������������������������������������������������Ŀ

� radio   �   computer       �    radio    �   computer  �

��������������������������������������������������������Ĵ

�A00-A49  � 000 - 049        �    a00-a49  � 050 - 099   �

��������������������������������������������������������Ĵ

�B00-B49  � 100 - 149        �    b00-b49  � 150 - 199   �

��������������������������������������������������������Ĵ

�C00-C49  � 200 - 249        �    c00-c49  � 250 - 299   �

��������������������������������������������������������Ĵ

�D00-D49  � 300 - 349        �    d00-d49  � 350 - 399   �

��������������������������������������������������������Ĵ

�E00-E49  � 400 - 449        �    e00-e49  � 450 - 499   �

��������������������������������������������������������Ĵ

�F00-F49  � 500 - 549        �    f00-f49  � 550 - 599   �

��������������������������������������������������������Ĵ

�G00-G49  � 600 - 649        �    g00-g49  � 650 - 699   �

��������������������������������������������������������Ĵ

�H00-H49  � 700 - 749        �    h00-h49  � 750 - 799   �

��������������������������������������������������������Ĵ

�I00-I49  � 800 - 849        �    i00-i49  � 850 - 899   �

��������������������������������������������������������Ĵ

�J00-J49  � 900 - 849        �    j00-j49  � 950 - 999   �

����������������������������������������������������������

VFO STEP VALUE.

This  display  shows  the current step  value that is used for

Search  mode. This value will be  added to or subtracted from the

VFO frequency to obtain the next VFO frequency. This value can be

set  using  the  left  (increment)  or  right  (decrement)  mouse

buttons, or direct entry in the field under the display. During a

bank search, this field will be loaded from the selected bank.

USE STD FREQ.

If this button is selected, frequencies  used during  a search

operation are  selected from  a  list of standards. Any frequency

that is not found in  the current list is not used in the search.

The standards list is DBASE compatible  file  that has a list  of

THE DISPLAY  10

RADIO REMOTE CONTROL PROGRAM FOR OS/2

valid frequencies. A sample file (STANDARD.DBF) is included. This

file  contains most  of  the  common  public  service frequencies

currently assigned by the FCC. The file can be modified using any

DBASE compatible  editor. A different  file  can also be selected

from the FILE menu.

PRIORITY INDICATOR

When  the  unit  is  scanning,  and  the  PRIORITY  button  is

selected, a specific channel is defined  as the priority channel.

When this  channel  is being scanned,  the  Priority Indicator is

lit. This is a red "P" situated next to the Channel # display.

DIGIT SCROLLING

The VFO Step Value, VFO  frequency, and  Channel # displays can

be changed using  the  left and right  mouse buttons to increment

and  decrement the digit. The  mouse pointer selects the digit to

change, and the left button will  increment, and the right button

will decrement the digit.  This  allows quick changing of  any of

the information in the display.

FREQ/MEMORY SCROLL BUTTONS

Both frequency and memory channel number  can be incremented or

decremented  using  the scroll  buttons  next  the  the  affected

display.  For  the Frequency, the increment  or  decrement  value

will  be  the current VFO  STEP setting. For  the memory  channel

number, increment or decrement value is always 1.

SIGNAL METER

The signal is a 12 step indicator corresponding  to  the signal

strength being detected. For ICOM and KENWOOD  radios, the signal

indicated  will always  be  9.  This is due to the fact that  the

signal detection  is a binary (On or  OFF) line,  with no  way to

read  the actual strength.  For  all  other  radios,  the  actual

strength detected by the radio will be  displayed.  For  a  LOCAL

radio, clicking  any  of the signal display buttons will turn  on

the signal at that strength. For all radios,  the OFF button will

temporarily turn  off  the signal. If scanning or searching, this

THE DISPLAY  11

RADIO REMOTE CONTROL PROGRAM FOR OS/2

will  cause  the  radio to  resume  scan  or  search.  For  LOCAL

operation, the OFF button is the only way to  turn off the signal

detection.

When not running in  LOCAL mode, the signal buttons can be used

to define a  minimum  squelch trigger. Clicking on  a button will

cause  the program to  ignore any signal at or  below that signal

level. When the squelch minimum is active, the signal limit level

buttons are displayed in green.

NOTE:This feature is not useful for the ICOM and KENWOOD radios,

as  these radios do not have  the  ability  to  determine  signal

strength values.

ATTENUATOR

The AOR3000 and AOR8000 have a built in attenuator. This can be

switched  on to reduce overload from  a  strong  signal. Pressing

this button will  toggle  the current attenuator setting  in  the

radio. When  in  SCAN  mode,  this  button  will  be  set by  the

currently scanning memory channel's attenuator setting.

SCAN MODE BUTTONS.

These buttons select the scanning mode. The program runs in one

of three scanning modes. MANUAL, SCAN  and SEARCH modes determine

the primary operation of the program. MANUAL mode is the default.

Manual Mode

Of three scanning modes. MANUAL mode  is the  default.  In this

mode, the program  will  continuously monitor the signal strength

from the radio. For the ICOM,  changes in the radio frequency  or

modulation will be logged in the VFO displays.

The  program will switch to manual mode if one of the following

conditions is met:

*   The MANUAL button is pressed.

*   SCAN mode detects no valid channels or banks.

*   SEARCH mode detects no valid  banks

THE DISPLAY  12

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Scan Mode

The  SCAN button starts the  program  scanning through selected

memory channels. Memory channels that are  flagged with the VALID

field will be scanned. Channels  with the VALID flag off  will be

bypassed.  When a signal is  detected, scanning  will  stop,  the

signal strength  will  be  displayed  in  the  Signal  Meter, and

information (duration, squelch count, etc) will be logged  in the

memory channel.  If the  MEM SYNC switch is selected, the  Memory

Channel display will change to the Memory  Channel number that is

being  logged.  When  the  signal  stops, scanning will continue.

Memory  channels  are normally incremented.  The direction of the

scan can be changed by  pressing the CHANNEL NUMBER scroll button

to the side of the channel display.

Several  parameters  and  memory  channel  settings  affect the

operation of SCAN. Each channel has a VALID  flag associated with

it. If this flag is  set  off, the corresponding  memory  channel

will  NOT be  scanned.  If the DELAY  field of the  corresponding

memory channel  is  set, the program will  delay  for a number of

seconds after the  signal stops  before resuming scan. The  DELAY

value can be set from the PARMS panel.

The SPEED CONTROL setting determines how fast each channel will

be scanned.  In the event that a  scan is  too fast to respond to

certain  frequencies, it can  be slowed by  adjusting  the  SPEED

CONTROL.  Setting the SPEED CONTROL to  a lower value  causes the

computer to wait on each channel when there is no signal present.

This gives the radio time for the squelch to detect a signal.

The  RESUME  switch  determines  the  action after the  squelch

breaks. If RESUME is  off, scanning will not  continue until  the

signal  ends. If  RESUME  is  on, scanning will resume  after the

number of  seconds specified by the RESUME  value. This value can

be set from the PARMS panel.

NOTE:The  RESUME  switch  overrides  any  RESUME setting  for an

individual memory channels.

If  a channel is entered in  the  PRIORITY  channel, it will be

scanned every  interval specified by  the  priority value if  the

PRIORITY switch is set.  If a signal is  detected on the priority

channel, normal  scanning  stops, and the program switches to the

priority channel. Normal scanning will resume  when the signal is

no longer  present  on the  PRIORITY  channel.  The  interval for

priority checking is set on the PARMS panel.

Note:The  RESUME counter  does not  function when stopped on the

priority  channel. Scan or Search will  resume  from  a  priority

channel ONLY when the signal disappears.

If BANKS ON  button  is on, the order of  channel  scan will be

determined by the START and END of  the current bank. The current

bank is  scanned from  0 to 99.  When  BANKS ON is selected, each

valid bank will determine starting and ending channels  for scan.

Channel numbers will be incremented for a  selected  bank  if the

START  channel  number is less  than  the  STOP  channel  number.

Otherwise, channel numbers will  be decremented.  Failure to find

THE DISPLAY  13

RADIO REMOTE CONTROL PROGRAM FOR OS/2

a  valid  bank,  or a valid channel for the specified banks  will

force MANUAL mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this  button  is selected for SCAN  or SEARCH

mode, the displayed Memory Channel information will change to any

channel  where   a  signal  is  detected.   This  allows  you  to

automatically  display  the  channel  information   whenever  the

squelch breaks.  If this button is  off, then the  memory channel

display will NOT change when a signal is detected.

The   COMPUTER/RADIO  switch   determines  the  source  of  the

frequency  information for  the  radio.  If COMPUTER is selected,

frequency and  modulation  information is sent to  the radio from

the computer's memory channels. If RADIO is set, then the program

will change the  radio's memory  channel  to  correspond  to  the

channel  being sent.  This  allows  use  of  the  radio's  memory

instead of the information in the computer. If RADIO is selected,

only the channel numbers supported by the selected  radio will be

allowed. The  maximum channel number  can  be  changed using  the

PARMS panel.

NOTE.The  OPTO  protocol  does  not  provide for memory  channel

control. When using the OPTO protocol, this switch remains in the

COMPUTER position, and cannot be changed.

Search Mode

Search  Mode  allows you to scan  a  range  of  frequencies for

action.  Up to 100 different ranges can be programmed. Each range

consists of a starting frequency,  stopping  frequency, increment

value  and  modulation. In  addition, searching can start at  the

current VFO frequency and be incremented or  decremented by a set

amount.  For  non-bank  processing,  the  program  will  normally

increment  the frequency. Pressing VFO  STEP  UP/DN  buttons will

change the search direction.

When  a  range is  searched, the  radio is  set  to the initial

frequency and modulation.  If a signal is detected, the frequency

and and modulation are recorded in the next free log  record, and

the  VALID flag  is  set for this  record.  If this frequency  is

already recorded in the log,  then COUNT, TIME  and SIGNAL values

are updated. If the log entry  has the VALID flag off, the signal

is ignored.  If there are  no more free  channels (frequency = 0)

then the message "SEARCH FULL" is displayed, and the  information

is NOT saved.

If the Use Std Freqs button is selected, the search frequencies

selected  are  adjusted to the  appropriate  standard  frequency.

Note that if BANK search is  selected,  and  the  Use  Std  Freqs

button is  selected, and no banks  have  a frequency range within

the standard database, the search will terminate.

THE DISPLAY  14

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Several  parameters  and  memory  channel  settings  affect the

operation  of  SEARCH. Each channel  has a VALID flag  associated

with it. If this flag  is  set off, the  corresponding  frequency

will NOT respond to a squelch break.  If the DELAY field of the a

memory channel is set, and the corresponding frequency breaks the

squelch, the program will delay for a number of seconds after the

signal  stops  before  resuming scan. The DELAY value can  be set

from the PARMS panel.

The  SPEED CONTROL setting determines how fast  each  frequency

will be monitored. In  the  event that  a search  is  too fast to

respond to certain frequencies, it can be slowed by adjusting the

SPEED CONTROL.  Setting the SPEED CONTROL to a lower value causes

the  computer  to wait  on each frequency when there is no signal

present. This gives the radio  time for the  squelch to detect  a

signal.

The  RESUME  switch  determines  the  action after  the squelch

breaks.  If RESUME  is off, searching will not continue until the

signal ends. If  RESUME  is  on,  scanning will resume after  the

number of seconds specified by the  RESUME value. This value  can

be set from the PARMS panel.

NOTE:The RESUME switch  overrides  any  RESUME  setting  for  an

individual memory channels.

If BANKS  ON  button is on, the order of frequency scan will be

determined by the START and END of the current bank.  When  BANKS

ON  is  selected,  each valid  bank  will determine starting  and

ending  frequency  ,  increment  and  modulation.  If  the  START

frequency  is  less than  the STOP frequency, then  the frequency

will   be   incremented.   Otherwise,  the  frequency   will   be

decremented.  Failure  to  find  a valid  bank will  force MANUAL

mode.

The MEM SYNC button allows the Memory Channel display to follow

active channels. If this button is selected, the displayed Memory

Channel information will change  to any channel where a signal is

detected. This  allows  you to  automatically display the channel

information whenever the squelch breaks.  If this button  is off,

then  the memory channel display will NOT change when a signal is

detected.

The  Use Std Freq button forces all frequencies within a search

range  to  be  set  only to  those  frequencies  in the  standard

database.  This  database is loaded  at startup time, and can  be

reloaded or changed using the FILE menu.

The PRIORITY button does NOT  have  any  effect on Search Mode.

Priority is  not  supported during a search operation.  Likewise,

the  COMPUTER/RADIO switch is  also  ignored.  Search Mode always

uses the computer's VFO.

THE DISPLAY  15

RADIO REMOTE CONTROL PROGRAM FOR OS/2

LOAD AND SET BUTTONS

Four  buttons provide load and set operations. These operations

transfer information  between  the  VFO,  memory channels and the

computer. When one of these  buttons is pressed, the current scan

mode  is suspended, and  the operation is started.  Pressing  the

button  a  second  time before the  operation  is  complete  will

terminate  the operation  and  resume the  suspended  scan  mode.

Selecting the MANUAL scan  button will also terminate any load or

set operation.

Load ALL

When  pressed,  all  of  the radio's memory channels are loaded

into the computer's memory channels. The load starts at the first

channel  in  the radio  (0  or  1)  and  continues to the maximum

channel.  The  maximum channel can be set on the PARMS panel. The

contents of computer memory channels are overwritten with the new

information.  Any  memory  channel  with a non-zero frequency  is

marked as valid. The  count,  duration  and  strength  fields are

reset to 0. The  service string  is also  cleared.  For  a  LOCAL

radio, a set of random frequency and  modulations is set for each

channel.

Note:  The OPTO  protocol does not  provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

Set  ALL

When pressed, the contents of the  computer memory channels are

stored  into the  radio's memory channels. The set starts at  the

first channel in the radio (0 or  1) and continues to the maximum

channel. The maximum channel  can be set on the  PARMS panel. The

contents of radio  memory channels are  overwritten  with the new

information. For ICOM and KENWOOD  radios,  a zero frequency will

cause the memory channel to be  cleared. For AOR3000 radios, zero

frequency channels will NOT be set.  For  LOCAL mode, each memory

channel will be transferred to the VFO in turn.

NOTE.The  OPTO protocol does  not  provide  for  memory  channel

control. When using the OPTO protocol, this switch is disabled.

THE DISPLAY  16

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Mem->VFO

This  button  will transfer  the contents of the Memory Channel

indicated   by  the  MEMORY  #  display,  to  the   VFO.  If  the

COMPUTER/RADIO switch is in the COMPUTER position, the Computer's

memory  channel  will  be transferred.  If the  RADIO position is

selected, the information  will be transferred from  the  radio's

memory. For a LOCAL  radio, the  input will be a random frequency

and modulation. This operation is the same as a Load All for  one

channel.

VFO->Mem

This button will transfer the contents of the VFO frequency and

modulation  to  the  Memory  Channel  indicated by  the MEMORY  #

display.  If  the   COMPUTER/RADIO  switch  is  in  the  COMPUTER

position, the Computer's memory channel  will be updated.  Count,

Strength,  and  Duration fields are reset. The Service string  is

also cleared.  If the RADIO position is selected, the information

will be  transferred to the radio's memory. For a LOCAL radio, no

data is transferred.  This  operation is the same  as a  Save All

for one channel.

OPERATION MODIFICATION BUTTONS

Next  to the  Channel # display  is a  column of  buttons  that

affect various operations. These buttons determine what memory is

used,  priority, resume,  bank selection, acknowledgment and  the

radio being used.  Each of these buttons is a toggle switch. Move

the mouse cursor over the button and right  click to toggle on or

off. The Priority  switch  has  an entry  field  next to it.  The

others have a status field.

COMPUTER/RADIO Button

For  various Scan modes and  operations, the  program needs  to

determine  who's  memory  is  the  source or  destination of  the

operation. When  in the COMPUTER position, the  program  is using

the internal  Memory  Channels  for  the operation.  If the RADIO

position is  selected,  the  operation  will  affect  the  memory

channels in the selected radio.

Note: The OPTO  protocol  does  not provide  for  memory  channel

THE DISPLAY  17

RADIO REMOTE CONTROL PROGRAM FOR OS/2

control. When using the OPTO protocol, this switch is disabled.

VFO Select Button

Above the Frequency display is a button used to select a VFO (A

or B) to  be  used in  the selected radio.  Some radios have  two

selectable frequency memories (VFO). This button determines which

VFO will be selected when setting the frequency on the radio.  If

the radio that is currently  connected does not support two VFOs,

then this button will not have any effect.

The MEM SYNC button allows the Memory Channel display to follow

active channels.  If  this button is selected for SCAN  or SEARCH

mode, the displayed Memory Channel information will change to any

channel  where  a  signal  is   detected.  This  allows   you  to

automatically  display   the  channel  information  whenever  the

squelch  breaks.  If  the  program  is in  MANUAL mode, then  the

memory channel  display  will  change  in  sync with  the  memory

channel number.

Priority

Priority scanning allows a  selected channel to be tested for a

signal  once  every interval. The channel number  to  select as a

Priority  channel  is  entered in  the entry field  next  to  the

Priority switch. If the  Priority switch is ON, then the priority

channel will be used.  The priority checking interval can  be set

using the PARMS panel. Priority only works during SCAN mode.

Banks On

When in SCAN  or SEARCH mode, the program can make use of banks

of  scan or search information. These banks allow the grouping of

channels or frequencies for  the SCAN mode. The  Banks  On switch

allows the scan mode  to use the  bank  associated  with the SCAN

mode.  When selected, the  indicator  next  to  the  switch  will

display what bank number (00-99) is currently being used.

THE DISPLAY  18

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Resume

The  Resume switch  will allow the  Scan or Search  to continue

when a signal is  present. This is useful when a  Scan  or Search

encounters  a  constant  signal. With  Resume  on, the  signal is

ignored  after  a  set amount of  time. This time interval is set

using the  Parms panel. The  indicator next  to the Resume switch

shows  approximately  how  many  seconds   are   left  until  the

SCAN/SEARCH resumes.

Note:This switch is  a global setting. If  on, the RESUME switch

for each memory channel is ignored.

Ack

Ack sets  radio acknowledgement on.  When a  command is sent to

the radio, the program will normally wait for acknowledgement  of

that command  to  come  from  the  radio. In  the event that  the

command is lost or garbled, the program will resend  the command.

Ack  forces the program to  wait  until  this  acknowledgement is

received  before  continuing  to the next  channel during SCAN or

SEARCH  mode.  If  Ack is OFF, then the program will not wait for

acknowledgment.  This will speed up some  Scan/Search operations,

but  may cause incorrect data to  be exchanged between the  radio

and program.

NOTE: If Ack  is ON, the radio will retry a  command up to three

times if  acknowledgement is set on. After the third failure, the

program will continue to the next command.

Logging

When  this switch is selected, all actions are logged to a data

file. This includes mode changes and squelch breaks. A time stamp

is  also logged with each event. This file is  cumulative, and is

not  erased by this program.  The name of the logging file can be

changed with the FILE menu selection.

Radio Selection

Up to  eight different  radios can be defined  to  the program.

These radios can have different BAUD rates, Com ports, protocols,

etc.  Each radio is assigned a name. You can change the name (and

other parameters) for a given radio from  the  Parms panel. Radio

Selection is  a drop-down selection  field.  Click  on the  right

button to  display  the radio list.  Double click  on a  radio to

THE DISPLAY  19

RADIO REMOTE CONTROL PROGRAM FOR OS/2

select it. The Baud and  Com port used will be  displayed next to

the Radio Selection control.  NOTE: When a new radio is selected,

Manual mode is forced.

Speed Control

For SCAN or SEARCH mode, the speed control determines the time

between  each channel or frequency. When the control  is all  the

way to the left, the time between each setting is minimal. If the

slider  is moved to the  right,  the program  will  delay  for  a

certain time before changing  the  setting  on  the radio. During

this  time, the program will monitor the radio for  a signal. The

SPEED control allows you to reduce  the  SCAN or SEARCH speed  to

give the radio more time to respond to a signal.

MEMORY  CHANNEL DATA FIELDS

Program  Memory  channels  contain information  for a  specific

radio setting.  Each  channel  contains information on Frequency,

Modulation and  attenuation to be sent  to (or received from) the

radio.  In addition,  information  on signal  strength, number of

squelch breaks and the frequency assignment are also displayed.

Each program channel  displayed  has  several fields associated

with it. Some  of these  fields are entry fields that information

can be  entered.  Other  fields are  toggle switches  that can be

turned  on and off.  Note  that entry  field  information  is NOT

accepted until ENTER is pressed, a  TAB  key  is  pressed, a  new

entry field is selected with the mouse, or the channel display is

changed with the scroll bars.

CHANNEL number

This  is  the first field  on  the display. It determines which

memory  channel  is  being  displayed  or   changed.  The  memory

channel can be changed  by direct  entry in the channel field, or

using  the  scroll  bars to the left  of the channel number.  The

PgUp  and PgDn keys will  also  scroll the memory channel  up and

down.  If entering the channel number, keep in mind that the data

will NOT be entered unless you press ENTER or a TAB key.

THE DISPLAY  20

RADIO REMOTE CONTROL PROGRAM FOR OS/2

VALID (Vld)

The VALID field determines whether or not SCAN and  SEARCH will

use the frequency  currently entered into that channel.  For SCAN

mode, the program will not scan the memory channel if VLD is OFF.

For the SEARCH operation, a frequency entered  in a memory record

that has the VALID flag off will NOT cause the search to stop and

log.

DELAY (Dly)

The DELAY  field is use  during the SCAN and SEARCH operations.

If  delay is set ON, scanning will continue DELAY  seconds  after

the signal  ends  on that  channel.  If  delay  is  set OFF, then

scanning will  continue  immediately after the  signal ends.  The

DELAY time value can be set on the PARMS panel.

ATTENUATION (Att)

The  ATTENUATION  field selects  the  attenuator of  the radio.

This is a circuit that reduces  the signal strength so  that high

level signals do not overload the  radio. The AOR3000 and AOR8000

radios support this  feature.  This  option  has  no  effect  for

KENWOOD and ICOM radios.

RESUME (Rsm)

The RESUME field selects the RESUME function for the individual

memory channel.  If this switch is  on, and a signal  is detected

for this memory  channel  in SEARCH or SCAN  mode, SEARCH or SCAN

will continue after a set number of second if  the signal has not

gone away. This works the same as the global RESUME switch except

that it only affects the single channel.

THE DISPLAY  21

RADIO REMOTE CONTROL PROGRAM FOR OS/2

FREQUENCY

The FREQUENCY field is used to record or set the current memory

channel  frequency.  Frequency  information  is  entered in  Mhz.

Note that INS and DEL keys do not work when editing the FREQUENCY

field.

MODE

The  MODE  field  represents  the  modulation  being used for a

specific frequency. This  field  is a drop down selection box. To

select a modulation, click on the button to the right of the box.

Double click  on  the  desired  modulation.  Note  that  not  all

modulations will be accepted by your particular radio.

COUNT

This  field  displays the number of times a signal was detected

on  a  specific channel.  This field  cannot be edited.  However,

left-clicking on this field with the mouse  cursor will reset the

field to 0.

TIME

This field displays the average number of seconds that a signal

was detected on a specific channel. This field cannot  be edited.

However, left-clicking on this field  with the mouse cursor  will

reset the field to 0.

SIGNAL

The  signal strength of  the last recorded signal  is  reported

here.  This field cannot  be  edited.  However,  left-clicking on

this field with the mouse cursor will reset the field to 0.

THE DISPLAY  22

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SERVICE

This  field  is  used to  record  the  service for  a  specific

channel.  This  is a  scratch pad location  to record information

about  a  specific  frequency.  If  the squelch  breaks  on  this

channel during  SCAN  or SEARCH mode, the program will attempt to

lookup the service for the frequency entered. If the frequency is

found in the current frequency database, and the SERVICE field is

blank, then it  will  be  filled in with the service found in the

database.

TIME STAMP

When ever a squelch  break  occurs  for  this channel, the time

stamp is updated with  the time and date of the occurrence.  This

field cannot  be  edited.  However,  left-clicking on this  field

with the mouse cursor will reset the field to 0.

SCAN BANK  DATA FIELDS

Scan  Bank entry  allows you  to define banks  of channels  for

scanning.  RADIOCTL allows you to assign any range of channels to

various banks. Up to 100 banks (0-99) can be assigned.  Banks can

overlap, and  can  be turned on or off selectively.  In order for

SCAN mode to recognize banks, the Banks On button must be active.

The bank displayed has several fields associated with it.

CHANNEL number

This is the first  field on the display.  It  determines  which

scan bank  is  being displayed or changed.  The  range  for  this

channel number is 0 to 99.  The Channel number can be  changed by

direct entry in  the  channel field, or using the scroll bars  to

the  left  of the channel number.  The  shift-PgUp and shift-PgDn

keys  will  also  scroll  the  memory channel  up  and  down.  If

entering the channel number, keep in  mind that the data will NOT

be entered unless you press ENTER or a TAB key.

THE DISPLAY  23

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Valid

The VALID field  determines whether or not SCAN  will use  this

bank.  Scanning banks without the VALID  flag will be ignored  at

all times.

Start

START  identifies the first channel  to start scanning with for

the given bank. When this bank is active, START will be the first

channel  scanned  if  the  channel's  VALID  flag  is set. If the

channel's VALID flag is NOT set, then  the  first  channel  to be

scanned will be the next sequential channel with a VALID flag.

STOP

STOP  identifies  the last channel  to scan when  the  bank  is

active.  After this channel  is  scanned, scanning  will continue

with the START channel of the next VALID scan bank.

If the STOP  channel  is set lower than the START channel,  the

SCAN will  decrement channel numbers.  If  STOP  is  greater than

START, channel numbers  will  increment. If  STOP  and  START are

equal,  the  single  channel number  in  START and  END  will  be

scanned.

SERVICE

This  field is provided  to  enter comments about a given bank.

It is not used or modified by any program function.

THE DISPLAY  24

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SEARCH BANK DATA FIELDS

The search bank  panel  allows  you to define banks of channels

for searching.  RADIOCTL  allows  you  to  assign  any  range  of

frequencies,  along  with  a  modulation  and   step  value   for

searching.  Up  to 100 search  banks (0-99)  can be  defined.  In

order for  SEARCH  mode to recognize  banks,  the Banks On button

must be active.  The bank displayed has several fields associated

with it.

When  searching, the program will start  with the first  search

channel marked as valid, set  the radio  VFO  to the frequency in

the  START field and the  modulation in the MODE field, and check

for  a  signal.  If  no  signal  is  detected,  the  program will

increment or decrement the  frequency  by the amount in  the  INC

field, and repeat the process.  When  the  frequency  in the STOP

field is reached,  the  program will select the next valid search

bank and repeat the process. When all valid search  channels have

been processed, the program returns to the first search bank. For

more details on the SEARCH operation, see the description of  the

SEARCH mode.

CHANNEL number

This is the first field on  the  display.  It  determines which

search bank  is  being displayed or changed.  The  range for this

channel number is 0 to 99.  The Channel number  can be changed by

direct entry  in the channel field,  or  using the scroll bars to

the left  of  the channel number.  The alt-PgUp and alt-PgDn keys

will also scroll the memory channel up and down.  If entering the

channel number,  keep in mind that  the data will NOT be accepted

unless you press ENTER or a TAB key.

VALID

The VALID field determines whether or not SEARCH will use  this

bank.  Search banks without the VALID flag will be ignored at all

times.

SEARCH BANK DATA FIELDS  25

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Start-Freq

Start-Freq  identifies  the  first  frequency  to  set  for the

search.  Please note that the program does NOT check the range of

this  value. If it is outside the range of  the radio,  the radio

may  ignore the setting.

Stop-Freq

Stop-freq  identifies the last frequency to set for the search.

The search bank ends when the vfo frequency reaches the Stop-freq

value.  If the Stop-freq value is less than the Start-freq value,

the frequency  is decremented. If the Stop-freq value is  greater

than the Start-freq value, the frequency is  incremented.  Please

note that  the program does NOT check the range of this value. If

it is  outside the range of the radio, the radio may  ignore  the

setting.

Incr-Freq

Inc-Freq determines the  steps  for the given search bank.  The

Inc-Freq value is added to (or subtracted from) the VFO frequency

for each step.  If an  Inc-Freq value of 0 is entered, the SEARCH

mode will automatically reset the VLD flag for that bank.

Mode

The MODE field  represents the modulation being  used  for  the

search  range.  The radio will be set  to  this modulation at the

beginning  of  the  search  range.  This  field  is  a  drop down

selection box. To select a modulation, click on the button to the

right of the  box. Double click on  the desired modulation.  Note

that  not  all modulations  will  be accepted by  your particular

radio.

SEARCH BANK DATA FIELDS  26

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Service

This  field is provided  to  enter comments about a given bank.

It is not used or modified by any program function.

STATUS LINE

The status line displays messages from the program. This may be

error messages, or normal  operation messages.  The time and date

are included with all messages as an indication of when the error

occurred.

SPECTRUM DISPLAY

The  spectrum  display shows  a graph  of  frequency vs. signal

strength.  An active channel will be shown as a red  line (if the

graph is  in the  range  of  the  active  frequency). Frequencies

representing past activity will be displayed in green.

In  the  lower left hand corner, the starting frequency will be

displayed. The ending frequency of the spectrum will be displayed

in  the  lower right  hand  corner  of the  display.  The  center

frequency of  the  spectrum is changed  by moving the  horizontal

scroll bar. The minimum start frequency is 0  mhz and the maximum

stop   frequency  is  9999.9mhz.  The  horz  step  increment   is

determined by the spectrum resolution.  The resolution is changed

by  moving the vertical  scroll bar. Resolution  goes from 100khz

per pixel to 1khz per pixel in 1khz steps.

Tic marks are displayed for reference.  Tic marks are displayed

every 10 mhz when the resolution is  above 10khz/pixel.  When the

resolution  is 10khz or below,  the tic marks are displayed every

1mhz.

SEARCH BANK DATA FIELDS  27

RADIO REMOTE CONTROL PROGRAM FOR OS/2

THE MENU

The menu on the top of the panel provides additional functions.

these functions provide the ability  to load and save information

to disk,  customize your radio setup, and  manipulate  the memory

channel  information. Menu items can be  selected by  placing the

mouse  cursor over the item and pressing the right  mouse button.

indicated by an underline under  the character  in  the  specific

menu  item.  For  the  primary  menu,  the  ALT key must  also be

pressed.  For  drop-down menus, only the key itself  is  need  to

select an item.

FILE MENU

The File menu  provides the ability to load or save the current

memory channel, scan banks, and  search banks to  or  from a text

file.  The  file  format for this  file is described in the  DATA

FORMAT section.  The scan information  can be  saved to or loaded

from many  different files. When a file  is  loaded,  only  those

memory channels, scan bank channels,  or search channels that are

included in the  file  are  changed.  For  a  save,  only  memory

channels with a frequency greater than zero , and scan and search

banks  that  are marked  as  valid are written.  If  the  program

detects a file format problem when loading, the type of error and

the line number causing the error will be displayed, and the load

will stop.

A  new DBASE  file  can also  be selected from this panel. This

file is used  for  service lookup operations. The format  of this

file is also discussed in the DATA FORMAT section.

A  new  STANDARD database file is set from the FILE menu.  This

database  is used during  SEARCH operations to force selection of

frequencies from this list. The format of this file is  discussed

in the DATA FORMAT section.

The name of the logging file is also set  from this menu.  This

is the file that is written to when the LOGGING switch is on.

The File menu also contains the Quit for  the program. Function

key 3 will also terminate the program.

The Menu  28

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DATABASE MENU

The Database menu allows you  to manipulate  various aspects of

the memory channels. A sort of memory channel by frequency can be

done from this  panel. With  a sorted frequency list,  the  radio

being controlled requires less  settling  time for each frequency

change, allowing for faster signal detection.

The  Service search  will  update  all  Service fields for  any

memory channel with a non-zero frequency. The service information

is obtained from the current DBASE file.

The Clear option allows you to clear all  memory channels,  all

service  fields from all memory channels, or all count,  time and

signal logs from all memory channels.

COMMON PARMS PANEL

Various parameters determine the operation of the program.  For

the  most  part, these parameters  are set once  when  you  first

install the program.  The first page of the Parms Notebook allows

you to set program operation parameters.

Priority

This  common  panel  parm  sets  the time  (in seconds) between

priority checks.

Delay

The number of seconds that the program  will  wait on a channel

after  a  signal  disappears. This value  only has affect if  the

DELAY flag is set on the current memory channel.

The Menu  29

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Resume

If RESUME is selected, this is the number of seconds that  SCAN

or SEARCH will continue after a signal is detected.

Spectrum Background

This  is  the color of the SPECTRUM DISPLAY window  background.

The default is BLACK. However, it can be changed to suit personal

preference.

Tone Decode Enable

This  selection is for a  future enhancement.  It  is currently

disabled, and cannot be selected.

RADIO  PARMS PANEL

The  program  provides settings for up to  eight  radios.  Each

radio can be setup with different operation characteristics.  For

the most part, these  parameters are  set  once  when  you  first

install  the  program. However, you  may  need  to  update  these

parameters if you connect different radios.  Except for Baud rate

and Com port, the defaults for the  particular radio types should

only be changed if a non-standard radio is being supported.

Radio Name

This  is  the  name that  you  give  the  radio.  There  are no

restrictions  on the  name,  other  than  is  is  limited  to  30

alpha-numeric  characters.  This  name   is  used  in  the  Radio

Selection control on the main panel.

The Menu  30

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Radio Type

This  field  defines  the type of  radio (protocol).  There are

currently six types of  radios supported. ICOM, KENWOOD, AOR3000,

AOR8000,  OPTO  and  LOCAL.  Other  radios  may  be added  in the

future.

TXADR

ICOM  and OPTO radios  require an address.  For the R-7000, the

address is normally 8. If you connect  a different ICOM  or  OPTO

radio, or you have more  than one  ICOM radio  connected, you can

change  this  address to match. This parameter has  no affect for

other radio types.

Max Channel

This field defines  the maximum  memory channel in  the  radio.

For  the ICOM R-7000, and KENWOOD R-5000, this would normally  be

99. For the AOR3000 this value should be 399. Changing this field

allows the  program to support other radios  with  more (or less)

memory.  This  parm has no effect for the  OPTO protocol, as  the

OPTO protocol does not support memory channels.

ORIGIN

Origin  is  the  first memory  channel. ICOM radios  start with

memory  channel  1.  The  AOR3000 and  KENWOOD start with  memory

channel 0.  This parm has no effect for the OPTO protocol, as the

OPTO protocol does not support memory channels.

MAX FREQUENCY

This is the maximum frequency that the radio will accept.

The Menu  31

RADIO REMOTE CONTROL PROGRAM FOR OS/2

MIN FREQUENCY

This is the minimum frequency that the radio will accept.

BAUD RATE

The baud setting  for this radio. Most radios can be configured

to accept baud rates from 1200 to 9600 baud.  The default setting

for ICOM  and AOR3000 is 9600.  For  the KENWOOD,  the default is

4800 baud. Adjust this setting to  match  the baud rate that your

radio is set for.

COM PORT

This  is  the  com  port  that  your radio is attached  to. The

program allows you to select  com  1 thru com 4 or LOCAL. A LOCAL

selection will bypass all serial port output.

DATA BITS

Most of the radios supported work with an 8 bit data format. If

you have a radio that requires 7 bits, this can be changed.

STOP BITS

Most  of the radios  supported use 2 stop  bits. If needed, you

can change this to 1 stop bit.

FLOW CONTROL

Flow   Control  provides   the  ability   to  stop  and   start

communications based on the ability of the radio  and computer to

accept data.  Flow control requires the use of additional control

lines (RTS/DTS).  These lines are used as  signals to indicate to

the radio and the computer to start and stop sending information.

The  ICOM  interface that this program is designed to  work  with

The Menu  32

RADIO REMOTE CONTROL PROGRAM FOR OS/2

does NOT provide for  RTS/CTS control  lines  (flow control). The

KENWOOD and AOR3000 interfaces will use RTS/CTS  flow control. It

is recommended that flow control be left on for these  radios, as

they may not work properly with it off.

SIGNAL DETECT

There are three  ways to detect a squelch  break.  The first is

using a spare signal on the RS-232 port. This signal detection is

part  of  the  modified interface,  and  also provided with  OPTO

equipped radios. This is referred to as HARDWARE detection.

The  second detection  method involves using  a  command to the

radio to  detect  squelch break.  This is referred to as SOFTWARE

detection. This method  also uses  a  command  to  the  radio  to

determine  the  signal  strength.   SOFTWARE   detection  is  NOT

available for the ICOM protocol.  Some KENWOOD radios  provide  a

software signal  detection.  The KENWOOD R5000 does NOT have this

feature.

The third method is a combination  of  HARDWARE  and  SOFTWARE.

With  this  technique,  the signal  line is checked  for  squelch

break,  and  a signal strength command  is  issued  ONLY  if  the

squelch  break signal has been detected. This method  is the most

efficient for scanning speed. However, it cannot be used for ICOM

radios, as there are no signal strength commands available.

Please note  that to make  use of the HARDWARE, or HARDWARE and

SOFTWARE  detect methods, either  the modified  interface  or the

OPTO interface must be present.

SEND DELAY

Some  radios  require a delay between each character  sent.  If

this value is set greater than 0, the computer will pause for the

number of milliseconds entered between each character sent to the

radio.  Although  this  delay  is  not  needed on  the  currently

supported  radios, this option is provided for future support  of

other radios.  The maximum  value that the program will allow for

this value is  50. If a greater value is set, it  will be reduced

when  the  PARMS  panel is  exited.  Be aware  that  setting this

parameter  will  slow  down  the  SCAN  and  SEARCH rates of  the

program.

NOTE:  The  delay time generated  is approximate. This is due to

the    unpredictability    of   the   multi-tasking   dispatching

environment. If this option is required, some experimentation may

be needed to determine the correct value.

The Menu  33

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The Menu  34

RADIO REMOTE CONTROL PROGRAM FOR OS/2

SPECIAL KEYS

ENTER

The  ENTER  key finalizes input to the  program. For data entry

fields, the information is stored into the computer's memory, and

the cursor advances to the next field.

ESC

Pressing ESC terminates the current operation. It is equivalent

to pressing the MANUAL button.

tab keys

The  tab keys will move the cursor to the next field. A forward

or reverse tab key will complete data entry of the current field.

PgDn/PgUp

These  keys increment or decrement the current  memory  channel

being displayed.

ctl-PgDn/ctl-PgUp

These keys increment or decrement the current scan bank channel

being displayed.

SPECIAL KEYS  35

RADIO REMOTE CONTROL PROGRAM FOR OS/2

alt-PgDn/ctl-PgUp

These  keys increment  or  decrement  the  current  search bank

channel being displayed.

SPECIAL KEYS  36

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DATA FILE FORMATS

There are several files that this  program makes use  of. These

files are used to store and retrieve data used by the program.

LOGFILE format

When  LOGGING is enabled, certain transactions are logged to a

file. The  name of this  file is  set by the  user.  The  initial

setting for this filename is RADIOCTL.LOG.

For a given transaction, the following information is logged:

*   Timestamp- The Date and Time of the transaction.

*   The scanning mode (SCAN/SEARCH/MANUAL).

*   The VFO frequency.

*   The VFO modulation.

*   The current memory channel.

*   The signal strength at the time of transaction.

*   The length of the transaction.

*   The service information for the current memory channel.

FREQLIST.DBF

This  is  a standard DBASE III+ type  data  file.  It  contains

frequencies and  allocations  for the  radio  spectrum. There are

three fields that are used by this program.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

DESCRIPT       character 50     usage of the frequency

ACTION         logical          flags the frequency in use

If the ACTION field is  not present,  the program will  use the

DATA FILE FORMATS  37

RADIO REMOTE CONTROL PROGRAM FOR OS/2

DESCRIPT  field  to determine  ACTION.  A  blank  DESCRIPT  field

signals an inactive channel.

The length of the DESCRIPT field can be changed, if so desired.

This program only uses the first 80 chars of this field.

Any other fields can be added or deleted from this file without

affecting the operation of  the program.  However, if  either the

FREQUENCY or  DESCRIPT field is  not present in the database, the

DBASE function will not operate.

STANDARD.DBF

This  is a  standard DBASE  III+  type  data file.  It contains

frequencies allocated  by  the FCC for two way  radio usage. This

list is NOT complete, but  can be added  to (or deleted from)  as

needed. Only one field is  defined and used. Other fields can  be

added, but the FREQUENCY field must NOT be deleted.

field name     allocation       description

-----------------------------------------------------------

FREQUENCY      decimal 10.5     holds the frequency in Mhz

Note that the frequencies contained  in this file are sorted in

memory after being loaded.  The load process can be speeded up by

pre-sorting  this  file  (low  frequency to high frequency).  The

original file included with the RADIOCTL package has already been

sorted.

data file format

The  SAVE command creates  a file with the current settings  of

memory channels, and various scan parameters. The file created is

an  ASCII  readable file. Each record is  terminated  by a  CR/LF

sequence. One record represents  one frequency channel, one  scan

bank channel, one search bank channel, one command, or a comment.

Which  type  if  record is  determined  by  the  first  non-blank

character on the line.  The following  table shows the identifier

for the various types of records:

DATA FILE FORMATS  38

RADIO REMOTE CONTROL PROGRAM FOR OS/2

��������������������������������������������Ŀ

� identifier   �  type of record             �

��������������������������������������������Ĵ

� number       � frequency channel info      �

��������������������������������������������Ĵ

� @            � scan bank information       �

��������������������������������������������Ĵ

� #            � search bank information     �

��������������������������������������������Ĵ

� keyword      � command to program          �

��������������������������������������������Ĵ

� *            � comment                     �

����������������������������������������������

Each record contains one or more fields delimited by blanks.  The

file can be altered (or  created) using any standard text editor.

The  maximum  record  length  that the program  can handle is 200

characters.  If  the  comment  record  flag   is   detected,  all

information following the '*' is ignored on that line.  When  the

program reads the file, any missing required parameter, parameter

range error, or unrecognized keyword will terminate the file read

at that line.

The format for a frequency channel entry is as follows:

DATA FILE FORMATS  39

RADIO REMOTE CONTROL PROGRAM FOR OS/2

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � CHANNEL   � 0-9999         �channel number to set       �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   2 � FREQUENCY � 0-999.9999     �frequency in Mhz            �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   3 � MODE      � 3 chars        �valid modulation for radio  �

�     �           �                � required-positional        �

���������������������������������������������������������������Ĵ

�   4*� COUNT     � 0-999999       �count of squelch breaks     �

���������������������������������������������������������������Ĵ

�   5*� DURATION  � 0-999999       �total duration of squelch   �

���������������������������������������������������������������Ĵ

�   6*� SIGNAL    � Sxx            �Signal strength value (xx)  �

�     �           � xx=00 to 15    � optional                   �

���������������������������������������������������������������Ĵ

�   7*� VALID     � VALID/NOTVALID �indicator of channel valid  �

���������������������������������������������������������������Ĵ

�   8*� ANTEN     � ANTEN/NOANTEN  �indicator of attenuation    �

���������������������������������������������������������������Ĵ

�   9*� RESUME    � RESUME/NORESUME�indicator of resume         �

���������������������������������������������������������������Ĵ

�  10*� TSTAMP    � TSTAMP value   �squelch break time stamp    �

�     �           � TSTAMP 0       �time value is UNIX time     �

�     �           �                �(number of seconds since    �

�     �           �                � 1970) 0 = no stamp.        �

���������������������������������������������������������������Ĵ

�  11*� TONE      � PL 0 0 0       �future enhancement.         �

�     �           �                �If included, all three      �

�     �           �                �numbers must be specified.  �

�     �           �                �                            �

���������������������������������������������������������������Ĵ

�  12*� DELAY     � DELAY/NODELAY  �indicator of channel delay  �

���������������������������������������������������������������Ĵ

�  13*� SERVICE   � 80 chars       �service for channel         �

�����������������������������������������������������������������

*optional fields

The first  three fields MUST be specified and be in  the proper

order. Optional fields do NOT have to be specified. However,

Optional  fields  do  not  have to be  specified.  However, the

following positional restrictions apply:,

*   If COUNT is specified, it MUST follow the MODE field.  If not

specified, the COUNT value for the channel is set to 0.

*   If  DURATION is specified,  COUNT must also be specified, and

DURATION  must  follow  the  COUNT.  If  not  specified,  the

DURATION value for the channel is set to 0.

*   Duration  value  MUST  be specified if  any  of the following

DATA FILE FORMATS  40

RADIO REMOTE CONTROL PROGRAM FOR OS/2

fields are specified:

SIGNAL, VALID, ANTEN, RESUME, TSTAMP, TONE, DELAY, SERVICE.

*   If  SIGNAL value  is not specified, then SIGNAL value for the

channel is set to 0.

*   If VALID  is not specified, the channel will be marked VALID.

*   If  ANTEN, RESUME  or DELAY is not specified,  the respective

flags will be reset for the channel.

*   If DELAY is specified, it MUST precede the service field.

*   If  SERVICE is specified, DELAY must  also be specified.  All

characters (up to 80) following the DELAY value and up to the

end  of the  line (CR/LF) are  taken  as  a  SERVICE  string.

Leading blanks and any special ASCII characters are removed.

DATA FILE FORMATS  41

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a scan bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � @              �@ identifies a scan bank    �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- MAX         �starting channel for bank   �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- MAX         �ending   channel for bank   �

���������������������������������������������������������������Ĵ

�   5*� SCAN      � SCAN/NOSCAN    �valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSCAN is not specified, the bank is marked as valid.

DATA FILE FORMATS  42

RADIO REMOTE CONTROL PROGRAM FOR OS/2

The format for a search bank entry is as follows:

���������������������������������������������������������������Ŀ

�field�  name     � value range    � description                �

���������������������������������������������������������������Ĵ

�   1 � BANKID    � #              �@ identifies a search bank  �

���������������������������������������������������������������Ĵ

�   2 � BANK      � 1-100          �bank number                 �

���������������������������������������������������������������Ĵ

�   3 � START     � 0- 9999.99999  �starting frequency for bank �

���������������������������������������������������������������Ĵ

�   4 � END       � 0- 9999.99999  �ending   frequency for bank �

���������������������������������������������������������������Ĵ

�   5 � INC       � 0- 9999.99999  �frequency increment         �

���������������������������������������������������������������Ĵ

�   5 � MODE      �AM,FM,FMN,etc.  �modulation for bank         �

���������������������������������������������������������������Ĵ

�   5*� SEARCH    � SEARCH/NOSEARCH�valid bank                  �

���������������������������������������������������������������Ĵ

�   6*� DESCRIPT  � 80 chars       �description of bank         �

�����������������������������������������������������������������

*optional fields

If NOSEARCH is not specified, the bank is marked as valid.

DATA FILE FORMATS  43

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Certain commands can be entered in a text  file  for execution.

These commands provide the ability  to execute certain operations

automatically, without  having to  select them  from the  various

panel controls.

COMMANDS RECOGNIZED

��������������������������������������������������������������Ŀ

�command         � option(s)      � description                �

��������������������������������������������������������������Ĵ

�ACK             �    -           �turn ON acknowledgement     �

��������������������������������������������������������������Ĵ

�BANK            � ON|OFF         �BANK mode on or off.        �

�                �                �Equivalent to pressing BANK �

�                �                �button.                     �

��������������������������������������������������������������Ĵ

�CHAIN           � filespec       �start reading another text  �

�                �                �file. Maximum CHAIN depth   �

�                �                �is 3 files.                 �

��������������������������������������������������������������Ĵ

�CHANNEL         � 0 to 9999      �set channel number.         �

�                �                �Equivalent to setting       �

�                �                �CHANNEL display value.      �

��������������������������������������������������������������Ĵ

�CLEAR           �    -           �Clear all memory channels   �

�                �    ALL         �Clear all memory channels   �

�                �    COUNTS      �Clear all squelch counts    �

�                �    SERVICE     �Clear all service strings   �

�                �    SIGNAL      �Clear all strength readings �

�                �    TIME        �Clear all squelch times     �

��������������������������������������������������������������Ĵ

�DBNAME          � filespec       �change DBASE filename for   �

�                �                �service lookups.            �

��������������������������������������������������������������Ĵ

�DELAY           �    xxxx        �Set DELAY value to xxxx.    �

��������������������������������������������������������������Ĵ

�F               �  frequency     �Set VFO frequency           �

��������������������������������������������������������������Ĵ

�HALT            �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�LOAD            �     -          �load memory from radio      �

��������������������������������������������������������������Ĵ

�M               �   modulation   �set VFO modulation          �

����������������������������������������������������������������

DATA FILE FORMATS  44

RADIO REMOTE CONTROL PROGRAM FOR OS/2

COMMANDS RECOGNIZED (cont)

��������������������������������������������������������������Ŀ

�NOACK           �     -          �turn OFF acknowledgement    �

��������������������������������������������������������������Ĵ

�MANUAL          �     -          �Force MANUAL mode           �

��������������������������������������������������������������Ĵ

�MEMORY          �     -          �use radio's memory          �

��������������������������������������������������������������Ĵ

�PDELAY          �   0 to 9999    �set priority sample delay   �

��������������������������������������������������������������Ĵ

�QUIT            �     -          �exit RADIOCTL               �

��������������������������������������������������������������Ĵ

�PRIORITY        �     ON         �set priority switch on      �

�                �     OFF        �set priority switch off     �

�                �   0 to 99999   �set priority channel        �

��������������������������������������������������������������Ĵ

�RESUME          �     ON         �set resume   switch on      �

�                �     OFF        �set resume   switch off     �

�                �   0 to 9999    �set resume value            �

��������������������������������������������������������������Ĵ

�SCAN            �     -          �Start SCAN mode             �

��������������������������������������������������������������Ĵ

�SEARCH          �     -          �Start SEARCH mode           �

��������������������������������������������������������������Ĵ

�SORT            �     -          �Sort memory channels        �

��������������������������������������������������������������Ĵ

�STD             �     ON         �set STD FREQ switch on      �

�                �     OFF        �set STD FREQ switch off     �

��������������������������������������������������������������Ĵ

�STDFILE         �   filename     �Specify STANDARD file name  �

��������������������������������������������������������������Ĵ

�VFO             �     -          �use computer's memory       �

����������������������������������������������������������������

For  compatibility  with  the  DOS  version  of  Radioctl,  the

following  command keywords  are  recognized,  but  ignored  when

reading:

*   BANK {bank_number}

*   BAUD

*   CD

*   COM1

*   COM2

*   DBASE

DATA FILE FORMATS  45

RADIO REMOTE CONTROL PROGRAM FOR OS/2

*   DIR

*   DMAX

*   DMIN

*   FILENAME

*   FKn

*   GOTO

*   HELP

*   INC

*   LOG

*   LOGNAME

*   LPT1

*   LPT2

*   MAX

*   MAXMEM

*   MODE

*   NOSIGNAL

*   PRINT

*   PRTLENGTH

*   PRTWIDTH

*   QFK

*   QSET

*   RECALL

*   RESET

*   SAVE

*   SET

*   SHELL

*   SIGNAL

*   SPEED

*   START

DATA FILE FORMATS  46

RADIO REMOTE CONTROL PROGRAM FOR OS/2

*   STOP

*   TXADR

Any  record that is not  recognized  as a Frequency, Scan  bank,

Search bank, command  or comment will be flagged  as  a  line  in

error.

DATA FILE FORMATS  47

RADIO REMOTE CONTROL PROGRAM FOR OS/2

RS232 INTERFACE

This program is designed  to use the serial communications port

(RS232)  of the  IBM computer.  In order to  interface a radio to

this  port, an  interface adapter  is  needed  designed  for  the

specific  radio. Each  radio manufacturer makes such  an adapter.

However,  none of the  adapters supplied  from  the  manufacturer

provide for squelch detection. Without this detection, the signal

sensing functions of this program will not work.

For  each  radio  supported,  a circuit  and  instructions  are

available.  This circuit provides the squelch detection needed by

this program.  The interface  circuits  are  easy  to build,  and

require few parts.  All  of  the parts can be obtained cheaply at

any electronics supply store. The schematics and instructions are

provided to all registered users.

RS232 INTERFACE  48

RADIO REMOTE CONTROL PROGRAM FOR OS/2

PROGRAM LIMITS AND RANGES

MEMORY REQUIRED:       4 meg (OS/2 minimum). 8 Meg+ suggested

PROCESSOR REQUIRED:    80386 or Higher

OS/2 LEVEL REQUIRED:   2.1  or higher

MAXIMUM MEMORY CHANNELS    10000

MAXIMUM SCAN BANKS          100

MAXIMUM SEARCH BANKS        100

FREQUENCY RANGE           .01khz to 9999.99999mhz*

MINIMUM FREQUENCY STEP     10hz*

MAXIMUM FREQUENCY STEP     9999.99999mhz

*may be limited by radio model.

PROGRAM LIMITS AND RANGES  49

RADIO REMOTE CONTROL PROGRAM FOR OS/2

INDEX

A                                  H

Ack 19                             Hot Keys

AOR3000 3                             Alt PgDn 25

attenuation 21                        Alt PgUp 25

Attenuator 12                         PgDn 20

PgUp 20

Shift PgDn 23

Shift PgUp 23

B

Banks On 18

I

ICOM 3

C                            interface 5

Channel # 9

channel RESUME 21

COMPUTER/RADIO Button 17                 K

COUNT 22

KENWOOD 3

D

L

DBASE 3

delay 20                           Load ALL 16

Digit Scrolling 11                 Load/Set Buttons 15

Logging 19

E

M

ENTER key 35

ESC key 35                         Manual Mode 12

Mem Sync 18

Mem->VFO 16

Memory Channel

F                               CHANNEL number 20

Memory Channels 20

Freq/Memory Scroll Buttons 11      Menu 27

frequency 21                          Database 28

FREQUENCY field 21                    File 28

Parms 29

MODE field 22

modulation 22

Index  50

RADIO REMOTE CONTROL PROGRAM FOR OS/2

Scan Mode

O                               BANKS 13

COMPUTER 14

Operation Modification Buttons 17     DELAY 13

MEM SYNC 14

PRIORITY 13

RADIO 14

P                               RESUME 13

SPEED 13

Parms                                 VALID 13

Baud Rate 32                    Search Bank 24

Com Port 32                        Channel number 25

Data Bits 32                       Incr-freq 26

Delay 29                           Mode 26

Flow Control 32                    Service 26

Max Channel 31                     Start-freq 25

Max Frequency 31                   Stop-freq 26

Min Frequency 31                   Valid 25

Origin 31                       Search Mode 14

Priority 29                        BANKS 15

Radio Name 30                      DELAY 15

Radio Type 30                      MEM SYNC 15

Resume 29                          RESUME 15

Send Delay 33                      SPEED 15

Signal Detect 33                   Use Std Freq 15

Spectrum Background 30             VALID 15

Stop Bits 32                    SERVICE 22

Tone Decode Enable 30           Set ALL 16

TXADR 31                        shareware 3

PgDn key 35                        SIGNAL 22

Pgup key 35                        Signal Meter 11

Priority 18                        Spectrum Display 27

Priority indicator 11

T

R

tab key 35

Radio Selection 19, 20             TIME  22

Resume 18                          TIME STAMP 22

RS232 5

U

S

Use Std Freq 10

Scan Bank

Channel number 23

Service 24

Start 24                              V

Stop 24

Valid 23                        valid 20

Scan Mode 13                       VFO Frequency 9

Scan Mode Buttons 12               VFO Modulation 9

MANUAL 12                       VFO Select Button 18

SCAN 13                         VFO step value 10

SEARCH 14                       VFO->Mem 17

Index  51