Notes about the sound drivers

��������������������������������������������������������������������������������

Driver Summary

Driver                  Def/Max Chan.   Stereo  Bits    Mixing Rate/Resolution

GUS, Hardwave           32/32           Yes     16      19->44kHz (A)

GUS, Software *         64/256          Yes     16

Interwave, Hardware     32/32           Yes     16      44kHz

Interwave, Software *   64/256          Yes     16

Sound Blaster 1.0       64/256          No      8       12kHz->22kHz

Sound Blaster 2.0       64/256          No      8       12kHz->44kHz

Sound Blaster Pro       64/256          Yes     8       6kHz->22kHz (B)

No      8       12kHz->44kHz (B)

Sound Blaster 16        64/256          Yes     16      12kHz->44kHz

Sound Blaster AWE 32 *  32/32           Yes     16      44kHz

Pro Audio Spectrum 16 * 64/256          Yes     16

PC Speaker              64/256          No      5-7     12->44kHz

DAC on LPT *            64/256          No      8       12->44kHz

Disk Writer             256/256         Yes     16      22->64kHz

Notes

A) Depends on number of channels used. The hiquality GUS driver reinitialises

the GUS continually to use as few channels as necessary.

B) The mixing rate of the SBPro depends on whether playback is stereo or mono

Drivers with * next to them have NOT been completed...

��������������������������������������������������������������������������������

Gravis UltraSound, Hardware mixing (ITGUS.DRV)

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s4 for Gravis UltraSound). This is equivalent to the original

internal driver that came with previous versions of Impulse Tracker.

The second driver is specified by providing the correct IRQ for the

GF1 chip. (The second-to-last number of your ULTRASND environment

variable). This is an IRQ driven routine, which means that it'll

work in the background of Windows '95. But note that the timing for

this is NOT as accurate as the timing in the first driver. There is

also a possibility that multitasking OSs can sometimes (although

rarely) cause some settings to the GUS to be missed (which will cause

a note to play unexpectedly). This can be fixed just by restarting

playback. There is NO check for the correctness of the IRQ provided.

The Gravis UltraSound *CANNOT* cope with 16-bit samples greater than

256k-bytes. This is equivalent to 128k-length samples. Also, 16-bit

samples cannot cross 256k boundaries on the GUS, meaning that the

amount of memory you have on the card may decrease by more than you

expect when you load a 16-bit sample.

You cannot choose the mixing rate for the GUS - the mixing rate is

dependent on the number of channels you initialise the program with

(using /Lxx)

Completed.

Gravis UltraSound 2, Hardware mixing (ITGUS2.DRV)

If the first Gravis UltraSound driver clicks continuously when nothing

is supposed to be playing, use this driver ("IT /sITGUS2.DRV").

This driver file also contains two drivers - check above on how to

access the second driver.

Completed.

Gravis UltraSound HiQuality driver, Hardware mixing (ITGUSHIQ.DRV)

OK. I have done something outrageous again. This driver is NOT

guaranteed to work on all GUSs. It works on my GUS MAX, tho...

It is also not guaranteed to provide continuous click-free playback,

although most of the time it operates smoothly.

This driver continuously reinitialises the GUS to as few channels

as required (between 14 and 32) to provide as high a mixing rate

as possible. This is NOT a standard procedure and it may well not

work on many GUSs (hope it does, tho!)

To run this driver, use "IT /sITGUSHIQ.DRV"

If you want to see how the GUS is being reinitialised, go to the

driver screen (Shift-F5) and enable reinitialisation info.

Completed.

Gravis UltraSound Mixed Driver

Not finished.

��������������������������������������������������������������������������������

InterWave Driver - Hardware mixing (ITIW.DRV)

(This includes GUS PnP, GUS PnP Pro, WavExtreme 32, WavExtreme 32 Pro + more)

You *NEED* to have RAM onboard to use this driver, otherwise your

Interwave card will NOT be detected.

This file actually contains two drivers in one file. The first is

accessed just by using IT (with no command line parameters, or with

/s6 for AMD Interwave IC). This is similar to the original internal

GUS driver that came with previous versions of Impulse Tracker.

The second driver is specified by providing the correct IRQ for the

Interwave chip. (This is the value given in Windows'95/settings/

control panel/system/Interwave SYNTH/IRQ). This is an IRQ driven

routine, which means that it'll work in the background of Windows '95.

But note that the timing for this is NOT as accurate as the timing in

the first driver. There is also a possibility that multitasking OSs

can sometimes (although rarely) cause some settings to the GUS to be

missed (which will cause a note to play unexpectedly). This can be

fixed just by restarting playback (or reinitialising in severe cases).

There is NO check for the correctness of the IRQ provided.

If you are running a multitasking OS, you MAY need to run IWINIT before

running Impulse Tracker to have more memory detected.

The Interwave driver contains handlers for two different memory modes

on the Interwave - the more memory efficient mode is where the amount

of ram is directly compatible with the interwave, the second is where

the DRAM configuration is NOT directly compatible with the interwave

and the driver has to handle the RAM slightly more explicitly, which

causes the loss of memory-usage efficiency.

Here are the modes directly compatible with the interwave:

Bank 0         Bank 1          Bank 2          Bank 3          Total

256Kb          0               0               0              256Kb

256Kb          256Kb           0               0              512Kb

256Kb          256Kb           256Kb           256Kb           1MB

256Kb          1MB             0               0             1.25MB

256Kb          1MB             1MB             1MB           3.25MB

256Kb          256Kb           1MB             0              1.5MB

256Kb          256Kb           1MB             1MB            2.5MB

1MB            0               0               0               1MB

1MB            1MB             0               0               2MB

1MB            1MB             1MB             1MB             4MB

4MB            0               0               0               4MB

* 4MB            4MB             0               0               8MB

* 4MB            4MB             4MB             4MB            16MB

* These modes cannot be handled by the first driver, so are actually

handled in the second mode.

The mixing rate for the Interwave driver is fixed at 44100Hz

(CD quality)

Bug warning: If the sound does NOT play properly, you may need to

run IWINIT before running Impulse Tracker

Completed.

Interwave Mixed driver

(This includes GUS PnP, GUS PnP Pro, WavExtreme 32, WavExtreme 32 Pro + more)

Not finished.

��������������������������������������������������������������������������������

Sound Blaster 1.0 driver (ITSB.DRV)

The Sound Blaster has a mixing range of 12000->21739 Hz. You CANNOT

hear any stereo (or surround) effects with this driver, because the

Sound Blaster does NOT support stereo.

Completed.

Sound Blaster 2.0 driver (ITSB2.DRV)

The Sound Blaster 2 driver is basically the same as the Pro driver

with stereo options removed... mixing rate from 12000->43478 mono.

Completed.

Sound Blaster Pro driver (ITSBPRO.DRV)

The Sound Blaster Pro has a mixing range of 12000->43478 in mono mode,

or 6000->21739 in stereo mode.

Hardware detection routines are used for Address (eg. 220h),

environment is checked for IRQ and DMA.

Command line arguments for IRQ and DMA are NOT checked but assumed

correct IF this driver is explicitly selected

(IT /s2 or IT /sITSBPRO.DRV).

Completed.

Sound Blaster 16 driver (ITSB16.DRV)

The Sound Blaster 16 has a mixing range of 12000->45454 in either mono

or stereo modes.

If you specify this driver ( IT /s3 or IT /sITSB16.DRV ) AND an IRQ

or DMA, IT will try to *FORCE* the SB16 to use this IRQ/DMA...

eg. On my system, I have my SB16 configured to IRQ 2, DMA 5, but I

can force it to use IRQ 7, DMA 0 with IT /s3 /i7 /d0

Hardware detection routines are used for all Address, IRQ and DMA.

Note: If you select either of the 32-bit mixing modes, then volumes

between 0->32768 are used internally instead of 0->128.

Completed.

��������������������������������������������������������������������������������

Sound Blaster AWE 32 driver (ITAWE32.DRV)

Not finished.

��������������������������������������������������������������������������������

PC Speaker (ITPCSPKR.DRV)

Notes: On the info page, using the 'variables' display WILL distort

PC Speaker output. Also, it has been found that the Info Page

screens and the Pattern Editor cause a noticeably higher amount

of hiss through the speaker.

The PC Speaker driver does NOT have its own driver screen.

Nothing much else to say here, except... GET A SOUND CARD! :)

Completed.

��������������������������������������������������������������������������������

Pro Audio Spectrum (PAS) 16

Not finished.

��������������������������������������������������������������������������������

DAC on LPT Driver

Not finished.

��������������������������������������������������������������������������������

Disk Writer

This WAV file writer is NOT available for public distribution.

Contact me if you wish to obtain this - it will NOT be made

available without some sort of (monetary) agreement.

Details

� 16 bit Stereo/Mono output

� 22kHz to 64kHz output frequency

� 16 bit interpolation (65536x 'oversampling')

� 32 bit mixing

� Software volume ramping (set at 500th of a second or less)

The disk writer also uses volumes ranging between 0 and 32768

instead of 0 to 128.

Completed.

��������������������������������������������������������������������������������