DEC 1994

LANsmart Installation

for

D-Link PCMCIA Ethernet Card

===========================

1. This installation procedure applies to the following D-Link products:

DE-650CT

*********************************** NOTE ***********************************

Each DE-650 series adapter comes with a driver diskette of its own.  If you

have more then one model of DE-650 adapter, make sure that you use the driver

diskette that was included with the DE-650 adapter you are using.

****************************************************************************

This diskette is labeled as LAN_DRV_DLK and the files are organized as follows:

Files          Contents

-----------    -------------------------------------------------

DE-65N.DRV     Driver program for the above-mentioned adapters.

DE-65N.EXE     Driver program for the above-mentioned adapters.

Note:

Before running LANSMART driver, run PCMCIA initial program on

A:\IBMPC\INSTALL directory. Please see as following:

*** PC installed Socket and Card services ***

Skip this step

OR

*** PC installed Socket service or Databook Cardtalk***

*** PC's PCMCIA controller is Intel 82365SL ***

*** PC is Toshiba T3300SL ***

DEVICE = DCD16.EXE

Please refer to \IBMPC\INSTALL\README.DOC for detail information.

2. Parameters for INSTALL utility:

DCD16.EXE

DCD16.EXE is an integration version of open adapter utility.

DCD16.EXE supports Intel 82365SL, Toshiba T3300SL's ICCNT, and Socket

Services v1.01, v2.0, v2.01, v2.10 Card Services v2.0, v2.01, v2.10.

For more detail information, please refer to \IBMPC\INSTALL\README.DOC.

Example:

device=DCD16.EXE [/iop:P0] [/irq:P1] [/slt:P2]

Parameter description:

/iop   :I/O base

/irq   :IRQ value

/slt   :Slot number

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable irq value.

The default value is 0x5.

P2 :The selectable slot number.

The default value is 0. You must set 0 when you use slot 1.

Set slt:1 when slot 2 is used,etc.

A. RUN LANsmart v.1.26 and v.1.30

==================================

Follow the procedure below to use the new LANsmart driver program on this

diskette with LANsmart v.1.26 and v.1.30.

1) Copy the new driver program DE-65N.DRV.

Example:

> COPY A:\IBMPC\LANSMART\DE-65N.DRV  C:\LANSMART\DE-65N.DRV

2) Now, you can use both the menu mode and the command mode to log onto the

network.  The LOGON command syntax is as follows:

> LOGON <username> <SRV|WKS> /NET:5 [other parameters]

B. RUN LANsmart version 2.xx & v3.xx

=====================================

You may use both the menu and the command mode to logon to the network.

The LOGON command syntax is as follows:

> LOGON <username> <PSV|WKS> /NET:DE-65N [other parameters]

The driver program DE-65N.DRV should first be copied to the directory that

contains LOGON.EXE program. /NET:DE-65N specifies to LANsmart your network

adapter card.