FUTURE DOMAIN CORPORATION

FDU-CD

CD-ROM Device Driver Version 2.30

I. GENERAL PRODUCT OVERVIEW:

1) Product description:

The FDU-CD CD-ROM device driver program is to be used with

Future Domain SCSI Host adapters to support one or more CD-ROM

drives on a DOS-based host system.  The device driver can support

from 1 to 24 CD-ROM drives (via up to 4 SCSI adapters) on the

PC, PC/XT, PC/AT and Micro Channel Architecture (PS/2) environments.

Photo-CD is supported on drives as noted in section 2.

2) Table of supported CD-ROM drives on 8 and 16 bit adapters:

MANUFACTURE	MODEL NUMBERS

-----------	----------------------------------

Chinon 	 CDX-435 CDS-435, CDC-435

Hitachi	 CDR-1750S, CDR-3650, CDR-3750

IBM		 CDROM I, CDROM II

LMSI		 CM-214

NEC		 CDR-25, CDR-72, CDR-73, CDR-74, CDR-83, CDR-84

(M-series are also supported)

Panasonic	 CR-301, CR-501, CR-532

Pioneer	 DRM-600

Sony		 CDU-541, CDU-561, CDU-6110, CDU-6111, CDU-6211,

CDU-7211

Texel		 DM-3024, DM-5024

Toshiba	 XME-3201B, XME-3301TA, XME-3401B

3) Table of Drives supporting Photo-CD:

MANUFACTURE	MODEL NUMBERS     SINGLE/MULTI-SESSION

-----------	-------------	  --------------------

NEC		 CDR-25		   Single-session

CDR-84		   Single-session

Panasonic	 CR-532		   Single-session

Sony		 CDU-541	   Single-session

CDU-561	   Single/Multi-session

Texel		 DM-3024/5024	   Single/*Multi-session

Toshiba	 XME-3301TA	   Single/*Multi-session

XME-3401B	   Single/Multi-session

(*) The asterisk specifies that the drive may require a

a Firmware and/or Hardware update for the specified Photo-CD

support.  Contact the Manufacture of the CD-ROM drive to verify

the drives Firmware revision if any problems occur.

II. SYSTEM REQUIREMENTS:

- Minimum 512KB of System Memory needed by INSTALL program.

(This amount of memory is required by the automatic installation

program - INSTALL.EXE.  Refer to "Manual Installation" in the

CD-ROM device driver Installation Manual Guide, if you cannot

run the automatic installation program.)

- MS or PC DOS Version 3.x - 5.0x.

- MS DOS Extensions Version 2.20/2.21 (may be included)

- A Hard Disk Drive. (Optional)

(Not necessary for device driver or host adapter but

may be necessary for some CD-ROM application programs.)

III. CHANGES FROM PREVIOUS RELEASE:

1) The following drive support has been added to this version of the

CD-ROM device driver:

MANUFACTURE	MODEL NUMBERS

-----------	----------------------------------

Chinon		 CDS-435, CDC-435

Panasonic	 CR-301, CR-532

Sony		 CDU-561

Texel		 DM-3024, DM-5024

Toshiba	 XM-3401B

IV. INSTALL/TECH TIPS (READ BEFORE INSTALLATION):

1) IMPORTANT - If the CD-ROM device(s) is attached to a Future Domain

SCSI adapter which contains a BIOS Version 3.00 or 8.00, you must

take one of the following actions:

a) Upgrade the BIOS.

b) Load the DOS CAM loadable driver. (DCAM950.EXE or DCAM1800.EXE)

c) Use the '/T' command line parameter to force the CD-ROM device

driver to talk directly to the SCSI adapter.

usage: DEVICE=C:\DEV\FDCD.SYS /D:MSCD000 /T

NOTE: The '/T' parameter must only be used with SCSI adapters that

contain the 3.00 or 8.00 BIOS.

Also, please do not configure your drives at SCSI IDs 6 or 7.

2) Command line parameter '/I' (controls Disk Change status)

If this switch is not specified, the driver will not return an

"Invalid Disc Change" error if the SAME disc is reinserted into

the drive.  This may cause a problem if two different disc's

having the same volume size are being used.

If you are experiencing this problem, specifying the /I switch

will force the driver to always return an "Invalid Disc Change"

error anytime a disc is changed. (SAME or DIFFERENT disc)

usage: DEVICE=C:\DEV\FDCD.SYS /D:MSCD000 /I

3) LASTDRIVE = Z in the CONFIG.SYS file.

The install program will default to using drive letter 'Z'.

This drive letter should be changed so that DOS does not reserve

more drive letters than are necessary.

example: If you have one fixed drive with two partitions using

drive letter C & D and one CD-ROM drive using drive

letter E, set LASTDRIVE to E (LASTDRIVE = E). This is

very important for Network users so that drive letters

will be available to access Network drives as in a

Novell environment.

4) You can remove the BIOS chip on the Future Domain SCSI controller

if there are no fixed drives attached.  This will allow the system

to boot-up much more quickly since the SCSI bus will not be scanned

by the BIOS upon boot-up.

5) For compatibility with the Future Domain FDDEVICE.SYS device

driver, load the FDDEVICE.SYS driver before loading FDCD.SYS

in the CONFIG.SYS file.

VI. TESTED SOFTWARE SUPPORT:

Microsoft:

- TESTDRV.EXE (CD-ROM device driver test program)

- CDSPEED.EXE (CD-ROM Data Streaming Transfer Rate Test)

- Windows 3.0 with Multimedia extensions V1.0

- Windows 3.1

- BookShelf

- Programmer's Library

- Musicbox with a variety of audio discs

- Multimedia Beethoven

National Geographic Society:

- Mammals: A Multimedia Encyclopedia

- The Presidents

Sierra Online:

- Mother Goose

Software Toolworks:

- CD Game Pack

- Illustrated Encyclopedia

- US Atals

- World Atlas

NOTE: The CD-ROM device driver will work under Windows 3.0/3.1 in REAL,

STANDARD and ENHANCED mode.  Depending on the application being

used, you may be limited to using REAL mode.  Contact the

manufacture of the application for compatibility under Windows.

VII. LIMITATIONS:

1) The following drives do not support Audio Channel Manipulation:

- Sony CDU-6110 (does not support audio)

- Sony CDU-6111 (supports stereo mode only)

VIII. CUSTOMER SUPPORT:

To contact our Customer Support Department please call :

Future Domain Corporation

2801 McGaw Ave.

Irvine, Ca. 92714

Telephone #: (714) 253-0440

Fax #:       (714) 253-0429

Monday - Friday, 8am - 5pm PST.