SCSIHA.SYS is a buffering device driver, and supports both video memory

and virtual memory buffering.  Video memory buffering can be used if trouble is

encountered running programs that transfer a lot of data directly from the disk

to video memory.  Applications that use the protected mode of the '386 (virtual

memory used by MS Windows 3.0) also require the use of SCSIHA.SYS.  Memory

managers such as QEMM and 386MAX also require SCSIHA.SYS.

SCSIHA.SYS is not required if using SMARTDRV.SYS or SMARTDRV.EXE with double

buffering enabled.  SCSIHA.SYS is also not required if using QEMM with double

buffering enabled (e.g. device=qemm.sys /db=2).

SCSIHA.SYS is only used with the AHA-1540/1640/1740 (standard mode) series

host adapters.

If ASPI4DOS.SYS is used, SCSIHA.SYS is not needed and should not be used.

SCSIHA.SYS should be loaded first in the CONFIG.SYS, and should not be loaded

high.

SCSIHA.SYS usage:

DEVICE=SCSIHA.SYS /V386 /B:xx

/V386   is used to enable virtual buffering.  If no /B:xx value is

specified, then a default of 16 kBytes is used.

/B:xx	 sets the buffer size where xx can be 16, 32, for 64 in units

of kBytes.

Example:

To run Windows 3.0 (only needed if SMARTDRV.SYS is not used),

the CONFIG.SYS should include the line

DEVICE=SCSIHA.SYS /V386

before HIMEM.SYS.