STANDARD MICROSYSTEMS CORPORATION

DEC PATHWORKS v4.1 for DOS

For TokenCard EliteSeries LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes how to:

o  Use EZStart to copy the DEC PATHWORKS version 4.1 for DOS

NDIS driver files from the SuperDisk to your PC.

o  Prepare your operating system so you can attach to the network.

Note: This driver supports all SMC TokenCard EliteSeries

ISA and Micro Channel Token-Ring adapters.

COPYING THE DRIVER FILES

From the Driver Installation screen, select 'Copy Files' to copy all

the files used by this driver from the SuperDisk to the Destination

Directory recommended by EZStart, or to a different directory that

you specify.

PREPARING YOUR OPERATING SYSTEM

1. Insert the DEC Pathworks Client Setup diskette #1 into your

computer's disk drive.

2. Type: NETSETUP

3. Press the Enter key.

4. When prompted about whether you have an LK250 keyboard, specify

'N(o)' if you do not have this type of keyboard.

5. Select the DECNET option and press Enter.

6. Insert the DNET TRNSPRT diskette into your computer's disk drive.

7. Specify the destination drive letter.

8. When the prompt 'Are you creating an initial workstation

diskette' appears, select N(o).

9. Type the node name and address.

10. Press Enter.

11. Type the server node name and number.

12. When prompted about whether you have an LK250 keyboard, specify

'N(o)' if you do not have this type of keyboard.

13. Select 'Token-Ring' and press Enter.

14. Select 'LAN Manager-Supplied NDIS'.

15. Press the Enter key twice.

16. Select 'IBM Token-Ring Adapter' and answer "No" when prompted

for Source Level routing. Then press Enter twice.

17. Press F9 to write the key disk. Then press Enter.

18. Modify the PROTOCOL.INI file as required. Make sure all the

values match the assigned values for the installed adapter.

When you finish, press Enter.

19. Follow the screen prompts.

20. After the Write Key Disk procedure is done, press F10 from the

Configuration screen to exit to DOS.

21. Copy the file SMC8100.DOS from the \NDIS subdirectory on the

SuperDisk to the \DECNET subdirectory on your computer's hard

disk.

22. Using any text editor, edit the CONFIG.SYS file by replacing

the line:

device=\DECNET\IBMTOK.DOS

with this line:

device=\DECNET\SMC8100.DOS

23. Use any text editor to make the following changes to the

PROTOCOL.INI file in the \DECNET subdirectory:

- Add the following lines:

[SMC]

Drivername = SMC8100$

- Change the BINDINGS= statements in the [IPX4MAC] and [DATALINK]

sections from:

IBMTOK.DOS

to:

SMC

- Remove the semicolon (;) in front of the NI_IRQ statement.

Make sure the ni_irq value in the [DATALINK] section matches

the irq value of the network adapter.

SMC and Standard Microsystems are registered trademarks; and

TokenCard, TokenCard Elite, TokenCard Elite/A, Elite Series,

EliteView, EZStart, and SuperDisk are trademarks of Standard

Microsystems Corporation.  Other products and company names are

registered trademarks or trademarks of their respective holders.